# Lab - OSPFv2 Route Summarization

# Topology

# Addressing Table

| Device | Interface | IPv4 Address |
| --- | --- | --- |
| R1 | E0/0 | 172.16.0.2/30 |
| R1 | E0/1 | 10.10.0.1/30 |
| R1 | Lo0 | 209.165.200.225/27 |
| R2 | E0/0 | 172.16.0.1/30 |
| R2 | E0/1 | 172.16.1.1/30 |
| R3 | E0/0 | 172.16.1.2/30 |
| R3 | E0/1 | 10.10.4.1/30 |
| D1 | E2/0 | 10.10.0.2/30 |
| D1 | E1/2 | 10.10.1.1/24 |
| D1 | Lo2 | 10.10.2.1/24 |
| D1 | Lo3 | 10.10.3.1/24 |
| D2 | E2/0 | 10.10.4.2/30 |
| D2 | E1/2 | 10.10.5.1/24 |
| D2 | Lo16 | 10.10.16.1/24 |
| D2 | Lo17 | 10.10.17.1/24 |
| D2 | Lo18 | 10.10.18.1/24 |
| D2 | Lo19 | 10.10.19.1/24 |
| D2 | Lo20 | 10.10.20.1/24 |
| D2 | Lo21 | 10.10.21.1/24 |
| D2 | Lo22 | 10.10.22.1/24 |
| D2 | Lo23 | 10.10.23.1/24 |
| PC1 | NIC | 10.10.1.10/24 |
| PC3 | NIC | 10.10.5.10/24 |

# Objectives

Part 1: Build the Network, Configure Basic Device Settings and Routing

Part 2: OSPFv2 Route Summarization

# Background / Scenario

Areas make OSPF more scalable and increase efficiency. Consider a large multinational organization with a thousand OSPF routers. If all routers were in a single area, the information contained in their LSDB would be overwhelming. Segmenting the OSPF domain into multiple areas reduces the size of the LSDB for each area, making SPF tree calculations faster, and decreasing LSDB flooding between routers when a link flaps.

To make OSPF even more scalable and efficient, network routes can be summarized and advertised in other areas. As well, specific route filtering can be used to provide more precise control on route propagation.

In this lab, you will configure route summarization and route filtering in a multiarea OSPF version 2 network. This lab was specifically designed to use three routers and two Layer 3 switches. To help visualize the potential of summarization and route filtering, additional loopback interfaces will be configured to simulate LANs and create larger routing tables.

Note: This lab is an exercise in developing, deploying, and verifying how OSPF route summarization and filtering operates and does not reflect networking best practices.

Note: Ensure that the routers and switches have been erased and have no startup configurations.

# Instructions

## Build the Network, Configure Basic Device Settings and Routing

In Part 1, you will set up the network topology and configure basic settings and interface addressing on the router and Layer 3 switches. You will also configure multiarea OSPFv2 on the OSPF backbone routers R1, R2, and R3. You will manually configure OSPFv2 on D1 and D2.

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

### Configure basic settings for the routers.

Console into each router, enter global configuration mode, and apply the basic settings, interface addressing, and OSPFv2 configuration. The configuration for each device is provided for you below.

- Note: Routers were configured with OSPFv2.

Open configuration window

Router R1

hostname R1

no ip domain lookup

line con 0

logging sync

exec-time 0 0

exit

banner motd # This is R1, OSPFv2 Route Summarization and Filtering Lab #

interface e0/0

ip add 172.16.0.2 255.255.255.252

no shut

exit

interface Ethernet0/1

ip address 10.10.0.1 255.255.255.252

no shut

exit

router ospf 123

router-id 1.1.1.1

auto-cost reference-bandwidth 1000

network 10.10.0.0 0.0.0.3 area 1

network 172.16.0.0 0.0.0.3 area 0

exit

Router R2

hostname R2

no ip domain lookup

line con 0

logging sync

exec-time 0 0

exit

banner motd # This is R2, OSPFv2 Route Summarization and Filtering Lab #

interface e0/0

ip add 172.16.0.1 255.255.255.252

no shut

exit

interface Ethernet0/1

ip address 172.16.1.1 255.255.255.252

no shut

exit

int lo0

ip add 209.165.200.225 255.255.255.224

exit

ip route 0.0.0.0 0.0.0.0 Loopback0

router ospf 123

router-id 2.2.2.1

auto-cost reference-bandwidth 1000

network 172.16.0.0 0.0.0.3 area 0

network 172.16.1.0 0.0.0.3 area 0

default-information originate

exit

Router R3

hostname R3

no ip domain lookup

line con 0

logging sync

exec-time 0 0

exit

banner motd # This is R3, OSPFv2 Route Summarization and Filtering Lab #

interface e0/0

ip add 172.16.1.2 255.255.255.252

no shut

exit

interface Ethernet0/1

ip address 10.10.4.1 255.255.255.252

no shut

exit

router ospf 123

router-id 3.3.3.1

auto-cost reference-bandwidth 1000

network 10.10.4.0 0.0.0.3 area 2

network 172.16.1.0 0.0.0.3 area 0

exit

Save the running configuration to startup-config.

Close configuration window

### Configure basic settings for the switches.

Console into the switch, enter global configuration mode, and apply the basic settings and interface addressing. A command list for each switch is provided below.

- Note: OSPF routing will be manually configured.

Open configuration window

Switch D1

hostname D1

no ip domain lookup

line con 0

exec-timeout 0 0

logging synchronous

exit

banner motd # This is D1, OSPFv2 Route Summarization and Filtering Lab #

interface e2/0

no switchport

ip address 10.10.0.2 255.255.255.252

no shut

exit

interface e1/2

no switchport

ip address 10.10.1.1 255.255.255.0

no shut

exit

int Lo2

ip add 10.10.2.1 255.255.255.0

ip ospf network point-to-point

exit

int Lo3

ip add 10.10.3.1 255.255.255.0

ip ospf network point-to-point

exit

Switch D2

hostname D2

no ip domain lookup

line con 0

logging sync

exec-time 0 0

exit

banner motd # This is D2, OSPFv2 Route Summarization and Filtering Lab #

interface e2/0

no switchport

ip address 10.10.4.2 255.255.255.252

no shut

exit

interface e1/2

no switchport

ip address 10.10.5.1 255.255.255.0

no shut

exit

int Lo16

ip add 10.10.16.1 255.255.255.0

ip ospf network point-to-point

exit

int Lo17

ip add 10.10.17.1 255.255.255.0

ip ospf network point-to-point

exit

int Lo18

ip add 10.10.18.1 255.255.255.0

ip ospf network point-to-point

exit

int Lo19

ip add 10.10.19.1 255.255.255.0

ip ospf network point-to-point

exit

int Lo20

ip add 10.10.20.1 255.255.255.0

ip ospf network point-to-point

exit

int Lo21

ip add 10.10.21.1 255.255.255.0

ip ospf network point-to-point

exit

int Lo22

ip add 10.10.22.1 255.255.255.0

ip ospf network point-to-point

exit

int Lo23

ip add 10.10.23.1 255.255.255.0

ip ospf network point-to-point

exit

Save the running configuration to startup-config.

Verify the interfaces configured on D1.

D1# show ip interface brief | include manual

Ethernet2/0  10.10.0.2       YES manual up                    up

Ethernet1/2  10.10.1.1       YES manual up                    up

Loopback2              10.10.2.1       YES manual up                    up

Loopback3              10.10.3.1       YES manual up                    up

- Notice the loopback interfaces configured on D1. Theses interfaces were configured for lab purposes to simulate other LANs.

- Note: Loopback interfaces were numbered based on the network address (e.g., Lo2 = 10.10.2.0/24) for convenience only.

Verify the interfaces configured on D2.

D2# show ip interface brief | include manual

Ethernet2/0  10.10.4.2       YES manual up                    up

Ethernet1/2  10.10.5.1       YES manual up                    up

Loopback16             10.10.16.1      YES manual up                    up

Loopback17             10.10.17.1      YES manual up                    up

Loopback18             10.10.18.1      YES manual up                    up

Loopback19             10.10.19.1      YES manual up                    up

Loopback20             10.10.20.1      YES manual up                    up

Loopback21             10.10.21.1      YES manual up                    up

Loopback22             10.10.22.1      YES manual up                    up

Loopback23             10.10.23.1      YES manual up                    up

- Again, notice the loopback interfaces configured on D1. Theses interfaces were configured for lab purposes to simulate other LANs.

Close configuration window

### Verify routing on R1, R2, and R3.

Verify the routing table of R1 using the show ip route ospf command.

Open configuration window

R1# show ip route ospf | begin Gateway

Gateway of last resort is 172.16.0.1 to network 0.0.0.0

O*E2  0.0.0.0/0 [110/1] via 172.16.0.1, 00:22:17, Ethernet0/0

10.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

O IA     10.10.4.0/30 [110/3] via 172.16.0.1, 00:21:43, Ethernet0/0

172.16.0.0/16 is variably subnetted, 3 subnets, 2 masks

O        172.16.1.0/30 [110/2] via 172.16.0.1, 00:22:11, Ethernet0/0

- The R1 routing table contains an OSPF internal or intra-area route, and interarea route, and an external route to the default gateway.

Verify the routing table of R2 using the show ip route ospf command.

R2# show ip route ospf | begin Gateway

Gateway of last resort is 0.0.0.0 to network 0.0.0.0

10.0.0.0/30 is subnetted, 2 subnets

O IA     10.10.0.0 [110/2] via 172.16.0.2, 00:19:40, Ethernet0/0

O IA     10.10.4.0 [110/2] via 172.16.1.2, 00:19:07, Ethernet0/1

- R2 is propagating the static default route and therefore does not have an external type 2 OSPF route (i.e., O* E2) in the routing table like R1 and R3.

Verify the routing table of R3 using the show ip route ospf command.

R3# show ip route ospf | begin Gateway

Gateway of last resort is 172.16.1.1 to network 0.0.0.0

O*E2  0.0.0.0/0 [110/1] via 172.16.1.1, 00:20:00, Ethernet0/0

10.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

O IA     10.10.0.0/30 [110/3] via 172.16.1.1, 00:20:00, Ethernet0/0

172.16.0.0/16 is variably subnetted, 3 subnets, 2 masks

O        172.16.0.0/30 [110/2] via 172.16.1.1, 00:20:00, Ethernet0/0

- Like R1, R3 has an internal route (LSA 2), an interarea route (LSA 3), and an external route (LSA 5).

- The LANs connected to D1 and D2 are not yet advertised.

Close configuration window

### Enable OSPFv2 on D1.

On D1, enable IP routing using the ip routing global configuration command.

Open configuration window

D1(config)# ip routing

Next, enter the OSPF router configuration mode using process ID 123, assign D1 the router ID 1.1.1.2 and set the reference bandwidth to distinguish between Gigabit Ethernet and FastEthernet interfaces.

D1(config)#

D1(config-router)#

D1(config-router)#

% OSPF: Reference bandwidth is changed.

Please ensure reference bandwidth is consistent across all routers.

- Note: Setting the reference cost value too high may cause issues with low-bandwidth interfaces.

Next, we need to have D1 advertise all four of its directly connected interfaces. Although this could be accomplished using four separate network statements, we will use the wildcard mask to advertise all four interfaces using one network statement.

D1(config-router)#

D1(config-router)#

*Mar  1 01:01:22.540: %OSPF-5-ADJCHG: Process 123, Nbr 1.1.1.1 on Ethernet2/0 from LOADING to FULL, Loading Done

Verify the OSPF routing table on D1.

D1# show ip route ospf | begin Gateway

Gateway of last resort is 10.10.0.1 to network 0.0.0.0

O*E2  0.0.0.0/0 [110/1] via 10.10.0.1, 00:05:20, Ethernet2/0

10.0.0.0/8 is variably subnetted, 9 subnets, 3 masks

O IA     10.10.4.0/30 [110/4] via 10.10.0.1, 00:05:20, Ethernet2/0

172.16.0.0/30 is subnetted, 2 subnets

O IA     172.16.0.0 [110/2] via 10.10.0.1, 00:05:20, Ethernet2/0

O IA     172.16.1.0 [110/3] via 10.10.0.1, 00:05:20, Ethernet2/0

Verify the routing table of R2 using the show ip route ospf command.

R2# show ip route ospf | begin Gateway

Gateway of last resort is 0.0.0.0 to network 0.0.0.0

10.0.0.0/8 is variably subnetted, 5 subnets, 2 masks

O IA     10.10.0.0/30 [110/2] via 172.16.0.2, 00:40:29, Ethernet0/0

O IA     10.10.1.0/24 [110/12] via 172.16.0.2, 00:06:56, Ethernet0/0

O IA     10.10.2.0/24 [110/3] via 172.16.0.2, 00:06:56, Ethernet0/0

O IA     10.10.3.0/24 [110/3] via 172.16.0.2, 00:06:56, Ethernet0/0

O IA     10.10.4.0/30 [110/2] via 172.16.1.2, 00:39:56, Ethernet0/1

- Notice how its routing table now includes routes to the D1 LANs. Notice also how this has increased the number of routing entries.

Close configuration window

### Enable OSPFv2 on D2.

On D2, enable IP routing using the ip routing global configuration command.

Open configuration window

D2(config)# ip routing

Next, enter the OSPF router configuration mode using process ID 123, assign D2 the router ID 3.3.3.2 and set the reference bandwidth to distinguish between Gigabit Ethernet and FastEthernet interfaces.

D2(config)#

D2(config-router)#

D2(config-router)#

% OSPF: Reference bandwidth is changed.

Please ensure reference bandwidth is consistent across all routers.

- Note: Setting the reference cost value too high may cause issues with low-bandwidth interfaces.

Advertise the 10.10.4.0/30 and 10.10.5.0 /255 networks. Again, this could be accomplished using separate network statements. However, the wildcard mask can be used to advertise both interfaces using one network statement as shown.

D2(config-router)#

D2(config-router)#

*Mar  1 01:15:02.643: %OSPF-5-ADJCHG: Process 123, Nbr 3.3.3.1 on Ethernet2/0 from LOADING to FULL, Loading Done

- Note: The wildcard mask 0.0.1.255 matches both networks 10.10.4.0/30 and 10.10.5.0/24

Next, advertise the 10.10.16.0/24 through to 10.10.23.0/24 loopback interface networks. Traditionally, this would require 8 network statements. But again, the wildcard mask can be used to advertise all 8 interfaces using one network statement as shown.

D2(config-router)#

D2(config-router)#

- Note: The wildcard mask 0.0.7.255 matches networks 10.10.16.0/24 through to 10.10.23.0/24.

Close configuration window

### Verify Routing.

    - Verify the routing table of D2 using the show ip route ospf command.

Open configuration window

D2# show ip route ospf | begin Gateway

Gateway of last resort is 10.10.4.1 to network 0.0.0.0

O*E2  0.0.0.0/0 [110/1] via 10.10.4.1, 00:02:19, Ethernet2/0

10.0.0.0/8 is variably subnetted, 24 subnets, 3 masks

O IA     10.10.0.0/30 [110/4] via 10.10.4.1, 00:02:19, Ethernet2/0

O IA     10.10.1.0/24 [110/14] via 10.10.4.1, 00:02:19, Ethernet2/0

O IA     10.10.2.0/24 [110/5] via 10.10.4.1, 00:02:19, Ethernet2/0

O IA     10.10.3.0/24 [110/5] via 10.10.4.1, 00:02:19, Ethernet2/0

172.16.0.0/30 is subnetted, 2 subnets

O IA     172.16.0.0 [110/3] via 10.10.4.1, 00:02:19, Ethernet2/0

O IA     172.16.1.0 [110/2] via 10.10.4.1, 00:02:19, Ethernet2/0

- D2 has OSPF route entries for:

One external OSPF route to the gateway of last resort.

The four D1 LANs (i.e., 10.10.0.0/30 through 10.10.3.0/24)

The two Area 0 networks (i.e., 172.16.0.0/30 and 172.16.1.0/30)

    - From R2, verify the routing table using the show ip route ospf command.

R2# show ip route ospf | begin Gateway

Gateway of last resort is 0.0.0.0 to network 0.0.0.0

10.0.0.0/8 is variably subnetted, 14 subnets, 2 masks

O IA     10.10.0.0/30 [110/2] via 172.16.0.2, 01:00:10, Ethernet0/0

O IA     10.10.1.0/24 [110/12] via 172.16.0.2, 00:26:37, Ethernet0/0

O IA     10.10.2.0/24 [110/3] via 172.16.0.2, 00:26:37, Ethernet0/0

O IA     10.10.3.0/24 [110/3] via 172.16.0.2, 00:26:37, Ethernet0/0

O IA     10.10.4.0/30 [110/2] via 172.16.1.2, 00:59:37, Ethernet0/1

O IA     10.10.5.0/24 [110/12] via 172.16.1.2, 00:09:55, Ethernet0/1

O IA     10.10.16.0/24 [110/3] via 172.16.1.2, 00:00:13, Ethernet0/1

O IA     10.10.17.0/24 [110/3] via 172.16.1.2, 00:00:13, Ethernet0/1

O IA     10.10.18.0/24 [110/3] via 172.16.1.2, 00:00:13, Ethernet0/1

O IA     10.10.19.0/24 [110/3] via 172.16.1.2, 00:00:13, Ethernet0/1

O IA     10.10.20.0/24 [110/3] via 172.16.1.2, 00:00:13, Ethernet0/1

O IA     10.10.21.0/24 [110/3] via 172.16.1.2, 00:00:13, Ethernet0/1

O IA     10.10.22.0/24 [110/3] via 172.16.1.2, 00:00:13, Ethernet0/1

O IA     10.10.23.0/24 [110/3] via 172.16.1.2, 00:00:13, Ethernet0/1

- Notice how the routing table of R2 now includes routes to the D1 and D2 LANs. And again, notice how this has increased the number of routing entries.

    - From D1, verify the routing table using the show ip route ospf command.

D1# show ip route ospf | begin Gateway

Gateway of last resort is 10.10.0.1 to network 0.0.0.0

O*E2  0.0.0.0/0 [110/1] via 10.10.0.1, 00:18:43, Ethernet2/0

10.0.0.0/8 is variably subnetted, 18 subnets, 3 masks

O IA     10.10.4.0/30 [110/4] via 10.10.0.1, 00:18:43, Ethernet2/0

O IA     10.10.5.0/24 [110/5] via 10.10.0.1, 00:09:56, Ethernet2/0

O IA     10.10.16.0/24 [110/5] via 10.10.0.1, 00:08:27, Ethernet2/0

O IA     10.10.17.0/24 [110/5] via 10.10.0.1, 00:08:27, Ethernet2/0

O IA     10.10.18.0/24 [110/5] via 10.10.0.1, 00:08:27, Ethernet2/0

O IA     10.10.19.0/24 [110/5] via 10.10.0.1, 00:08:27, Ethernet2/0

O IA     10.10.20.0/24 [110/5] via 10.10.0.1, 00:08:27, Ethernet2/0

O IA     10.10.21.0/24 [110/5] via 10.10.0.1, 00:08:27, Ethernet2/0

O IA     10.10.22.0/24 [110/5] via 10.10.0.1, 00:08:27, Ethernet2/0

O IA     10.10.23.0/24 [110/5] via 10.10.0.1, 00:08:27, Ethernet2/0

172.16.0.0/30 is subnetted, 2 subnets

O IA     172.16.0.0 [110/2] via 10.10.0.1, 00:18:43, Ethernet2/0

O IA     172.16.1.0 [110/3] via 10.10.0.1, 00:18:43, Ethernet2/0

Notice the OSPF routing table now includes the additional interarea routes from D2.

Close configuration window

## OSPFv2 Route Summarization

As shown in Part 1, routing tables increase in the number of entries as more and more networks are connected to the OSPF domain.

To reduce the size of the routing table and LSDB, network prefixes must be summarized. Route summarization improves OSPF performance as fewer network entries are required.

Route summarization involves consolidating multiple routes into a single advertisement. Proper route summarization reduces the bandwidth, memory, and CPU resources consumed by the OSPF process.

OSPF routes can only be summarized between areas. Interarea route summarization is configured on ABRs using the area area-id range network subnet-mask [advertise | not-advertise] [cost metric] router configuration command.

| Parameter | Description |
| --- | --- |
| - area area-id | - Identifies the area subject to route summarization. |
| - address | - The summary address designated for a range of addresses. |
| - mask | - The IP subnet mask used for the summary route. |
| - advertise | - Enabled by default, it sets the address range status to advertise and generate a type 3 summary LSA. |
| - not-advertise | - (Optional) Sets the address range status to DoNotAdvertise. - Can be used for route filtering as the type 3 summary LSA is suppressed, and the component networks remain hidden from other networks. |
| - cost cost | - (Optional) Metric or cost for this summary route, which is used during the OSPF SPF calculation to determine the shortest paths to the destination. - The value can be 0 to 16777215. |

In this part, you will learn how to reduce the number of routing entries without compromising access to any networks.

### Configure interarea route summarization on R1.

Area 1 consists of networks 10.10.0.0/30, 10.10.1.0/24, 10.10.2.0/24, and 10.10.3.0/24. To calculate the summary address of these networks:

List the networks in binary format.

Count the number of left-most matching bits to determine the mask.

Copy the matching bits and add zero bits to determine the network address.

- The four networks are listed in binary format.

| - Network | - 1st Octet | - 2nd Octet | - 3rd Octet | - 4th Octet |
| --- | --- | --- | --- | --- |
| - 10.10.0.0 | - 0000 1010 | - 0000 1010 | - 0000 0000 | - 0000 0000 |
| - 10.10.1.0 | - 0000 1010 | - 0000 1010 | - 0000 0001 | - 0000 0000 |
| - 10.10.2.0 | - 0000 1010 | - 0000 1010 | - 0000 0010 | - 0000 0000 |
| - 10.10.3.0 | - 0000 1010 | - 0000 1010 | - 0000 0011 | - 0000 0000 |

There are 22 left-most bits that match. Octet 1 and 2 match for a sum of 16 bits. There are 6 left-most bits that match in the 3rd octet which results in a total of 22 bits that match.

A /22 subnet converts to 255.255.252.0.

Therefore, the summary network address of networks 10.10.0.0/30, 10.10.1.0/24, 10.10.2.0/24, and 10.10.3.0/24 is 10.10.0.0 255.255.252.0.

Open configuration window

      - On the Area 1 ABR router R1, enter OSFP router config mode.

R1(config)# router ospf 123

Summarize the D1 LANs using the area 1 range 10.10.0.0 255.255.252.0 router configuration command.

R1(config-router)#

R1(config-router)#

Close configuration window

### Verify the interarea route summarization.

Verify the routing table of R1 using the show ip route ospf command.

Open configuration window

R1# show ip route ospf | begin Gateway

Gateway of last resort is 172.16.0.1 to network 0.0.0.0

O*E2  0.0.0.0/0 [110/1] via 172.16.0.1, 00:40:51, Ethernet0/0

10.0.0.0/8 is variably subnetted, 16 subnets, 4 masks

O        10.10.0.0/22 is a summary, 00:40:51, Null0

O        10.10.1.0/24 [110/11] via 10.10.0.2, 00:40:51, Ethernet0/1

O        10.10.2.0/24 [110/2] via 10.10.0.2, 00:40:51, Ethernet0/1

O        10.10.3.0/24 [110/2] via 10.10.0.2, 00:40:51, Ethernet0/1

O IA     10.10.4.0/30 [110/4] via 172.16.0.1, 00:40:51, Ethernet0/0

O IA     10.10.5.0/24 [110/4] via 172.16.0.1, 00:40:51, Ethernet0/0

O IA     10.10.16.0/24 [110/4] via 172.16.0.1, 00:40:51, Ethernet0/0

O IA     10.10.17.0/24 [110/4] via 172.16.0.1, 00:40:51, Ethernet0/0

O IA     10.10.18.0/24 [110/4] via 172.16.0.1, 00:40:51, Ethernet0/0

O IA     10.10.19.0/24 [110/4] via 172.16.0.1, 00:40:51, Ethernet0/0

O IA     10.10.20.0/24 [110/4] via 172.16.0.1, 00:40:51, Ethernet0/0

O IA     10.10.21.0/24 [110/4] via 172.16.0.1, 00:40:51, Ethernet0/0

O IA     10.10.22.0/24 [110/4] via 172.16.0.1, 00:40:51, Ethernet0/0

O IA     10.10.23.0/24 [110/4] via 172.16.0.1, 00:40:51, Ethernet0/0

172.16.0.0/16 is variably subnetted, 3 subnets, 2 masks

O        172.16.1.0/30 [110/2] via 172.16.0.1, 00:40:51, Ethernet0/0

- The routing table of the ABR using the summary address adds a discard route entry to the Null 0 interface to prevent routing loops. The portions of the summarized network range will not have a more specific route in the routing table.

Verify the routing table of R2 using the show ip route ospf command.

R2# show ip route ospf | begin Gateway

Gateway of last resort is 0.0.0.0 to network 0.0.0.0

10.0.0.0/8 is variably subnetted, 11 subnets, 3 masks

O IA     10.10.0.0/22 [110/2] via 172.16.0.2, 00:36:38, Ethernet0/0

O IA     10.10.4.0/30 [110/2] via 172.16.1.2, 02:20:03, Ethernet0/1

O IA     10.10.5.0/24 [110/12] via 172.16.1.2, 01:30:21, Ethernet0/1

O IA     10.10.16.0/24 [110/3] via 172.16.1.2, 01:20:39, Ethernet0/1

O IA     10.10.17.0/24 [110/3] via 172.16.1.2, 01:20:39, Ethernet0/1

O IA     10.10.18.0/24 [110/3] via 172.16.1.2, 01:20:39, Ethernet0/1

O IA     10.10.19.0/24 [110/3] via 172.16.1.2, 01:20:39, Ethernet0/1

O IA     10.10.20.0/24 [110/3] via 172.16.1.2, 01:20:39, Ethernet0/1

O IA     10.10.21.0/24 [110/3] via 172.16.1.2, 01:20:39, Ethernet0/1

O IA     10.10.22.0/24 [110/3] via 172.16.1.2, 01:20:39, Ethernet0/1

O IA     10.10.23.0/24 [110/3] via 172.16.1.2, 01:20:39, Ethernet0/1

- Notice how the previous four route entries for network 10.10.0.0 to 10.0.3.0 have now been summarized onto one route.

Close configuration window

### Configure interarea route summarization on R3.

Area 2 consists of networks 10.10.4.0/30 and 10.10.5.0/24. It also consists of LANs 10.10.16.0/24 through to 10.10.23.0/24. These networks are not contiguous and cannot easily be summarized. For this reason, two summary commands will be configured on R3.

Open configuration window

On R3 enter OSFP router config mode.

R3(config)#

The first summary advertisement will be for the 10.10.4.0/30 and 10.10.5.0/24 networks. To summarize they are listed in binary format.

| - Network | - 1st Octet | - 2nd Octet | - 3rd Octet | - 4th Octet |
| --- | --- | --- | --- | --- |
| - 10.10.4.0 | - 0000 1010 | - 0000 1010 | - 0000 0100 | - 0000 0000 |
| - 10.10.5.0 | - 0000 1010 | - 0000 1010 | - 0000 0101 | - 0000 0000 |

- There are 23 left-most bits that match. Octet 1 and 2 match for a sum of 16 bits. There are 7 left-most bits that match in the 3rd octet which results in a total of 23 bits that match.

- A /23 subnet converts to 255.255.254.0. Therefore, the summary network address of networks 10.10.4.0/30 and 10.10.5.0/24 is 10.10.4.0 255.255.254.0.

- Summarize the D2 LANs and using the area 2 range 10.10.4.0 255.255.254.0 router configuration command.

R3(config-router)#

The second summary advertisement will be for the 10.10.16.0/24 through to 10.10.23.0/24 networks. Although all eight networks could be listed in binary format, it is possible to discover the summary addresses by only listing the first network and last network in binary format.

| - Network | - 1st Octet | - 2nd Octet | - 3rd Octet | - 4th Octet |
| --- | --- | --- | --- | --- |
| - 10.10.16.0 | - 0000 1010 | - 0000 1010 | - 0001 0000 | - 0000 0000 |
| - 10.10.23.0 | - 0000 1010 | - 0000 1010 | - 0001 0111 | - 0000 0000 |

There are 21 left-most bits that match. Octet 1 and 2 match for a sum of 16 bits. There are 5 left-most bits that match in the 3rd octet which results in a total of 21 bits that match.

A /21 subnet converts to 255.255.248.0. Therefore, the summary network address of networks 10.10.16.0/24 through to 10.10.23.0/24 is 10.10.16.0 255.255.248.0.

A cost can also be assigned to a summary route by using the cost keyword.

Summarize the D2 LANs and assign them a cost of 65 using the area 2 range 10.10.16.0 255.255.248.0 cost 65 router configuration command.

R3(config-router)#

Close configuration window

### Verify the interarea route summarization

Verify the routing table of R1using the show ip route ospf command.

Open configuration window

R3# show ip route ospf | begin Gateway

Gateway of last resort is 172.16.1.1 to network 0.0.0.0

O*E2  0.0.0.0/0 [110/1] via 172.16.1.1, 00:01:14, Ethernet0/0

10.0.0.0/8 is variably subnetted, 14 subnets, 6 masks

O IA     10.10.0.0/22 [110/3] via 172.16.1.1, 00:01:14, Ethernet0/0

O        10.10.4.0/23 is a summary, 00:01:14, Null0

O        10.10.5.0/24 [110/2] via 10.10.4.2, 00:01:14, Ethernet0/1

O        10.10.16.0/21 is a summary, 00:01:14, Null0

O        10.10.16.0/24 [110/2] via 10.10.4.2, 00:01:14, Ethernet0/1

O        10.10.17.0/24 [110/2] via 10.10.4.2, 00:01:14, Ethernet0/1

O        10.10.18.0/24 [110/2] via 10.10.4.2, 00:01:14, Ethernet0/1

O        10.10.19.0/24 [110/2] via 10.10.4.2, 00:01:14, Ethernet0/1

O        10.10.20.0/24 [110/2] via 10.10.4.2, 00:01:14, Ethernet0/1

O        10.10.21.0/24 [110/2] via 10.10.4.2, 00:01:14, Ethernet0/1

O        10.10.22.0/24 [110/2] via 10.10.4.2, 00:01:14, Ethernet0/1

O        10.10.23.0/24 [110/2] via 10.10.4.2, 00:01:14, Ethernet0/1

172.16.0.0/16 is variably subnetted, 3 subnets, 2 masks

O        172.16.0.0/30 [110/2] via 172.16.1.1, 00:01:14, Ethernet0/0

- R3 added two discard route entries to the Null 0 interface to prevent routing loops.

Verify the routing table of R2 using the show ip route ospf command.

R2# show ip route ospf | begin Gateway

Gateway of last resort is 0.0.0.0 to network 0.0.0.0

10.0.0.0/8 is variably subnetted, 3 subnets, 3 masks

O IA     10.10.0.0/22 [110/2] via 172.16.0.2, 00:01:25, Ethernet0/0

O IA     10.10.4.0/23 [110/2] via 172.16.1.2, 00:01:25, Ethernet0/1

O IA     10.10.16.0/21 [110/66] via 172.16.1.2, 00:00:04, Ethernet0/1

- Notice how R2 now has only two route entries for the D2 LANs. Previous to route summarization on R3, R2 had 10 route entries for the D2 LANs. Also notice the cost of the 10.10.16.0/21 route has been influenced.

Verify the routing table of D1 using the show ip route ospf command.

D1# show ip route ospf | begin Gateway

Gateway of last resort is 10.10.0.1 to network 0.0.0.0

O*E2  0.0.0.0/0 [110/1] via 10.10.0.1, 01:19:46, Ethernet2/0

10.0.0.0/8 is variably subnetted, 10 subnets, 5 masks

O IA     10.10.4.0/23 [110/4] via 10.10.0.1, 00:10:04, Ethernet2/0

O IA     10.10.16.0/21 [110/68] via 10.10.0.1, 00:09:23, Ethernet2/0

172.16.0.0/30 is subnetted, 2 subnets

O IA     172.16.0.0 [110/2] via 10.10.0.1, 01:19:46, Ethernet2/0

O IA     172.16.1.0 [110/3] via 10.10.0.1, 01:19:46, Ethernet2/0

- Notice how its routing table is smaller.

    - Verify connectivity to D2.

D1# ping 10.10.23.1 source 10.10.1.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.10.23.1, timeout is 2 seconds:

Packet sent with a source address of 10.10.1.1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/2/8 ms

Close configuration window