# Lab – Implementing MPLS VPNs (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Topology

# Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R1 | E0/0 | 10.1.12.1 | 255.255.255.0 |
| R1 | E0/1 | 10.1.11.1 | 255.255.255.0 |
| R1 | Loopback 0 | 10.1.1.1 | 255.255.255.255 |
| R2 | E0/0 | 10.1.12.2 | 255.255.255.0 |
| R2 | E0/1 | 10.1.23.2 | 255.255.255.0 |
|  | Loopback 0 | 10.2.2.1 | 255.255.255.255 |
| R3 | E0/0 | 10.1.23.3 | 255.255.255.0 |
| R3 | E0/1 | 10.1.32.3 | 255.255.255.0 |
| R3 | Loopback 0 | 10.3.3.3 | 255.255.255.255 |
| D1 | E2/0 | 10.1.11.2 | 255.255.255.0 |
| D1 | Loopback 0 | 198.51.100.1 | 255.255.255.128 |
| D2 | E2/0 | 10.1.32.2 | 255.255.255.0 |
| D2 | Loopback 0 | 209.165.201.1 | 255.255.255.128 |

# Objectives

# •	Configure Open Shortest Path First (OSPF) and Enhanced Interior Gateway Routing Protocol (EIGRP) on a router

# •	Enable MPLS on a router

# •	Verify MPLS implementation

# •	Configure a Virtual Routing/Forwarding (VRF) instance

# •	Use MBGP to exchange VPN routing updates

# •	Verify VPN activity

# Background / Scenario

Scenario

As a network engineer at a service provider corporation, you suggest rolling out MPLS as a new transport technology to facilitate VPNs between customer sites that connect through your network. Your CIO has asked you to implement proof-of-concept in a lab environment, starting with a small implementation of MPLS VPNs before moving up to more moderately sized test cases.

MPLS VPN technology is a powerful technology that leverages the multiprotocol aspect of MPLS to switch MPLS frames between VPN endpoints while hiding the customer networks from the MPLS transport network that connects them. In other words, the intermediate transport network has no knowledge of the customer’s IP networks, but is still able to label-switch frames based on information it receives from MPLS Label Distribution Protocol (LDP) relationships.

You decide to model one of your current customer’s connections and then show how MPLS VPNs can be used to carry customer traffic through the provider network. The International Travel Agency currently uses your network to connect from its corporate headquarters to a remote D2 office, so you choose this customer network to model in your demonstration.

Set up the model of both the service provider’s network and the agency’s network. Then use appropriate routing and forwarding techniques to set up an MPLS VPN between the provider edge routers to which the customer connects. R1, R2, and R3 will represent a service provider network, and D1 and D2 will represent the International Travel Agency routers at their headquarters and at a D2 site.

Instructor Note: Refer to the Instructor Lab Manual for the procedures to initialize and reload devices.

# Required Resources

3 Routers (Cisco 4221 with Cisco IOS XE Release 16.9.4 universal image or comparable)

2 Switches (Cisco 3650 with Cisco IOS XE release 16.9.4 universal image or comparable)

1 PC (Choice of operating system with terminal emulation program installed)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

# Instructions

## Build the Network and Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings.

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

### Configure basic settings for each device.

Console into each device, enter global configuration mode, and apply the basic settings for the lab. Initial configurations for each device are listed below.

Open configuration window

Router R1

hostname R1

no ip domain lookup

line con 0

exec-timeout 0 0

logging synchronous

exit

interface e0/0

ip address 10.1.12.1 255.255.255.0

no shutdown

exit

interface e0/1

ip address 10.1.11.1 255.255.255.0

no shutdown

exit

interface loopback 0

ip address 10.1.1.1 255.255.255.255

no shutdown

exit

exit

end

Router R2

hostname R2

no ip domain lookup

line con 0

exec-timeout 0 0

logging synchronous

exit

interface e0/0

ip address 10.1.12.2 255.255.255.0

no shutdown

exit

interface e0/1

ip address 10.1.23.2 255.255.255.0

no shutdown

exit

interface loopback 0

ip address 10.2.2.1 255.255.255.255

no shutdown

end

Router R3

hostname R3

no ip domain lookup

line con 0

exec-timeout 0 0

logging synchronous

exit

interface e0/0

ip address 10.1.23.3 255.255.255.0

no shutdown

exit

interface e0/1

ip address 10.1.32.3 255.255.255.0

no shutdown

exit

interface loopback 0

ip address 10.3.3.1 255.255.255.255

no shutdown

exit

exit

end

Switch D1

hostname D1

no ip domain lookup

ip routing

banner motd

line con 0

exec-timeout 0 0

logging synchronous

interface ra e0/0-3,e1/0-3,e2/0-3

shutdown

exit

interface e2/0

no switchport

ip address 10.1.11.2 255.255.255.0

no shutdown

exit

interface loopback 0

ip address 198.51.100.1 255.255.255.128

no shutdown

exit

Switch D2

hostname D2

no ip domain lookup

ip routing

line con 0

exec-timeout 0 0

logging synchronous

exit

interface ra e0/0-3,e1/0-3,e2/0-3

shutdown

exit

interface e2/0

no switchport

ip address 10.1.32.2 255.255.255.0

no shutdown

exit

interface loopback 0

ip address 209.165.201.1 255.255.255.128

no shutdown

exit

end

Set the clock on all devices to UTC time.

Save the running configuration to startup-config on all devices.

Close configuration window

Step 3: Configure Routing in the Service Provider Domain

Your service provider network uses OSPF as its routing protocol, advertising internal loopback interfaces and transit networks. Configure OSPF to model the service provider domain. Add all of the interfaces addressed within the 10.0.0.0 major network into Area 0 of the OSPF process. You will only need to configure OSPF in this manner on the service provider routers, namely R1, R2 and R3.

R1(config)# router ospf 1

R1(config-router)# network 10.1.12.0 0.0.0.255 area 0

R1(config-router)# network 10.1.1.1 0.0.0.0 area 0

R2(config)# router ospf 1

R2(config-router)# network 10.1.0.0 0.255.255.255 area 0

R3(config)# router ospf 1

R3(config-router)# network 10.1.23.0 0.0.0.255 area 0

R3(config-router)# network 10.3.3.1 0.0.0.0 area 0

Verify that all of your OSPF adjacencies come up. OSPF adjacencies should form between R1 and R2 and between R2 and R3. If the adjacencies do not form, troubleshoot by checking your interface configuration, OSPF configuration, and physical connectivity.

What purpose does OSPF serve in the configuration above?

OSPF serves as the routing protocol that connects routers owned by the service provider (SP), providing transit service between R1, R2, and R3.

Consider that you will deploy Border Gateway Protocol (BGP) in the SP domain later in the lab using loopback addresses as the sources for BGP updates. Why do you need to deploy an Interior Gateway Protocol (IGP) in the SP domain?

If you attach the BGP update-source Loopback0 attribute to a neighbor statement, the BGP routing table on the adjacent router will use the address of the Loopback0 interface of the advertising peer as the next-hop address. In order for the BGP routes to be installed in the routing table, however, the BGP next-hop address—the peer’s loopback network—must also be installed in the routing table. IGPs, such as OSPF, facilitate the advertisement of the loopback networks so that the update-source attribute may be employed.

Step 4: Configure MPLS in the SP Domain

On all the service provider routers, force MPLS to use the Loopback 0 interface as the router ID for Label Distribution Protocol (LDP) adjacencies. The loopback interface would be chosen by each router automatically, but it is advisable to force the ID so that the value is persistent through topology changes and reloads. To force LDP’s selection of the loopback interface as the router ID, use the mpls ldp router-id interface force command in global configuration mode. Also, enable MPLS on all of the physical interfaces in the MPLS domain with the mpls ip command.

R1(config)# mpls ldp router-id loopback0 force

R1(config)# interface e0/0

R1(config-if)# mpls ip

R2(config)# mpls ldp router-id loopback0 force

R2(config)# interface e0/0

R2(config-if)# mpls ip

R2(config-if)# interface e0/1

R2(config-if)# mpls ip

R3(config)# mpls ldp router-id loopback0 force

R3(config)# interface e0/0

R3(config-if)# mpls ip

You should see console messages notifying you that the MPLS-enabled routers have become adjacent with each other via LDP. Verify that these adjacencies have formed using the show mpls ldp neighbor command.

R1# show mpls ldp neighbor

Peer LDP Ident: 10.2.2.1:0; Local LDP Ident 10.1.1.1:0

TCP connection: 10.2.2.1.36109 - 10.1.1.1.646

State: Oper; Msgs sent/rcvd: 10/11; Downstream

Up time: 00:00:57

LDP discovery sources:

Ethernet0/0, Src IP addr: 10.1.12.2

Addresses bound to peer LDP Ident:

10.1.12.2       10.1.23.2       10.2.2.1

R2#show mpls ldp neighbor

Peer LDP Ident: 10.1.1.1:0; Local LDP Ident 10.2.2.1:0

TCP connection: 10.1.1.1.646 - 10.2.2.1.36109

State: Oper; Msgs sent/rcvd: 13/12; Downstream

Up time: 00:02:39

LDP discovery sources:

Ethernet0/0, Src IP addr: 10.1.12.1

Addresses bound to peer LDP Ident:

10.1.12.1       10.1.11.1       10.1.1.1

Peer LDP Ident: 10.3.3.1:0; Local LDP Ident 10.2.2.1:0

TCP connection: 10.3.3.1.23058 - 10.2.2.1.646

State: Oper; Msgs sent/rcvd: 10/10; Downstream

Up time: 00:00:04

LDP discovery sources:

Ethernet0/1, Src IP addr: 10.1.23.1

Addresses bound to peer LDP Ident:

10.1.23.1       10.1.32.1       10.3.3.1

R3#show mpls ldp neighbor

Peer LDP Ident: 10.2.2.1:0; Local LDP Ident 10.3.3.1:0

TCP connection: 10.2.2.1.646 - 10.3.3.1.23058

State: Oper; Msgs sent/rcvd: 10/10; Downstream

Up time: 00:00:12

LDP discovery sources:

Ethernet0/0, Src IP addr: 10.1.23.2

Addresses bound to peer LDP Ident:

10.1.12.2       10.1.23.2       10.2.2.1

Step 5: Configure a VRF

An MPLS VPN is a Layer 3 VPN that allows the routing of packets through a MPLS core. This type of VPN provides a customer with connections to multiple sites through a service provider’s network. The service provider not only provides the physical connection, but the ability to dynamically route between the VPN endpoints. This is especially impressive when one considers that the customers may not be using globally unique Layer 3 addresses. For instance, different customers can use private addresses, as defined by RFC 1918, but still use the same transit provider to route their specific endpoints without translation. The routers at the provider’s edge run the same routing protocol as the customer’s network and allow the customer offices to interface with the provider.

The standard model for MPLS VPNs uses the following designations:

Provider (P) — Routers owned by the SP that act as label switch routers (LSRs) to provide transit across the provider backbone. P routers do not carry customer routes in their routing tables.

Customer (C) — Routers owned by the customer that provide transit through the normal customer network

Customer Edge (CE) — The CE router is installed at the customer site. Depending upon the business model of the ISP, this router may be managed by the customer, the ISP, or both. The CE router connects to, and communicates with, the service provider routers, and allows the service provider to participate in customer routing.

Provider Edge (PE) — Routers owned by the provider that actively participate in customer routing, guaranteeing optimum routing between customer sites. PE routers use a separate virtual routing table for each customer, resulting in perfect isolation between customers.

It is important to note that neither the C nor the CE routers need any special configuration. The P routers only require a simple MPLS LDP configuration.

In this lab, R2 models the P router, R1 and R3 model the PE routers. D1 and D2 are both CE routers with loopback networks to simulate connections to other C routers.

The PE routers control the entire MPLS VPN from end to end. There are a number of relevant questions you may be asking: How can a single router determine which routes in its table belong to the service provider’s internal network and which routes belong to each customer? How can the PE device allow customers to utilize existing networks including private addressing without creating routing problems?

The answer to all of these questions lies in the ability of routers to maintain virtual routing and forwarding (VRF) instances. Each VRF uses and maintains its own routing information base (RIB) and Cisco Express Forwarding (CEF) table. Interfaces are either assigned to specific VRF instances or they use the default RIB and CEF tables. The VRF instance’s RIB fulfills the role of control plane while the VRF’s CEF table fulfills the role of the data forwarding plane. Routing protocols between the PE and CE routers populate the VRF RIB and CEF makes forwarding decisions based on the routes in the VRF RIB. When an IP packet arrives on an interface that has been associated with a VRF, the packet is routed according to the CEF table for that VRF instance. CEF is the only IP switching protocol supported for VRF, so CEF should be enabled globally with the ip cef command and on the interfaces associated with the VRF instance.

However, PE routers must now be connected through the provider network to perform routing and forwarding between customer sites. The most efficient and only scalable method to achieve this is to use the multiprotocol extensions to BGP (MP-BGP) that enable it to carry routes for different routed protocols. PE routers will establish iBGP sessions with other PEs in your carrier network to exchange for each VPN routes. This will help populate the VRF routing tables on each of the PE routers with the VRF tables from other customer sites. CEF tables will be updated with the RIB information so that forwarding may occur between customer sites once the label-switched paths have been created through the provider network.

PE routers advertise routes that are part of their VPN using a new traffic class to distinguish these routes from internal routes in the provider’s network. BGP uses a new address family called VPNv4 to carry MPLS-VPN routes to IPv4 networks. The VPNv4 address family is a 12-byte address consisting of an 8-byte route distinguisher (RD) and a 4-byte IPv4 address. The RD acts as a unique prefix when appended with the IPv4 address. Each VRF must have an RD for unique advertisement.

VRFs use the route target attribute to control the import and export of VPNv4 routes through iBGP. The route target is an extended BGP community that indicates which routes should be imported from MP-BGP into the VRF. Exporting a route target (RT) means that the exported VPNv4 route receives an additional BGP extended community—this is the route target—when the route is redistributed from the VRF RIB into MP-BGP. Importing an RT means that the received VPNv4 route from MP-BGP is checked for a matching extended community—this is the route target—with the ones in the configuration.

To configure a VRF instance on the PE routers, use the ip vrf name command in global configuration mode on R1 and R3. At the VRF configuration prompt, create a VRF named “customer.” Each VRF instance will need a route distinguisher and a route target. The route distinguisher and route target are each eight bytes in length, with a colon separating four bytes on either side. There are various conventions for allocating route distinguishers for MPLS VPNs, the most useful of which is ASN:nn. Another popular notation is IP address:nn. In each of these cases, ‘nn’ represents an arbitrary value assigned by the network administrator. In this lab, use 100:1 as the route distinguisher. The route target is also an arbitrary eight-byte value used later in BGP.

Configure a route distinguisher (RD) of 100:1 and route target (RT) of 1:100 using the commands rd ASN:nn and route-target {import | export | both} nn:nn. In this case, you need to use the both keyword because you want PEs to import and export from that VRF.

R1(config)# ip vrf customer

R1(config-vrf)# rd 100:1

R1(config-vrf)# route-target both 1:100

R3(config)# ip vrf customer

R3(config-vrf)# rd 100:1

R3(config-vrf)# route-target both 1:100

Imagine that R1 is running MP-BGP and it receives a VPNv4 route with a route target of 100:100. Given the above configuration, should BGP import the route into the customer VRF routing table?

No, MP-BGP will only import VPNv4 routes into the customer VRF routing table if the route has a route-target of 1:100. No routes other than those with a route target of 1:100 will be imported.

After creating the VRFs, add interfaces to the VRF using the interface-level ip vrf forwarding name command, where name is the VRF instance name. Use this command on the interfaces of R1 and R3 (the PE routers) facing the CE routers. Add the IP address shown in the diagram to those interfaces, as well.

R1(config)# interface e0/1

R1(config-if)# ip vrf forwarding customer

R1(config-if)# ip address 10.1.11.1 255.255.255.0

R1(config-if)# no shut

R3(config)# interface e0/1

R3(config-if)# ip vrf forwarding customer

R3(config-if)# ip address 10.1.32.1 255.255.255.0

R3(config-if)# no shutdown

You should now be able to ping across those the PE-CE links because you configured the other end of these links in Step 1. However, since these are not in the default routing table, you must use the ping vrf name address command. Since the VRF is transparent to the customer routers, you can use a traditional ping command when you are pinging from the C and CE routers.

R1# ping vrf customer 10.1.11.2

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.1.11.2, timeout is 2 seconds:

.!!!!

Success rate is 80 percent (4/5), round-trip min/avg/max = 1/1/1 ms

D1#ping 10.1.11.2

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.1.11.2, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 4/4/4 ms

R3# ping vrf customer 10.1.32.2

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.1.32.2, timeout is 2 seconds:

.!!!!

Success rate is 80 percent (4/5), round-trip min/avg/max = 1/1/1 ms

D2#ping 10.1.32.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.1.32.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 4/4/5 ms

Step 6: Configure EIGRP AS 1

The service provider by whom you are employed uses the BGP AS 100. Your customer, the International Travel Agency, uses the BGP AS 1. To keep the configuration logically consistent, use the AS number 100 for EIGRP and BGP in the provider’s network and use the AS number 1 for EIGRP and BGP in the customer’s network. You will configure EIGRP AS 1 on the PE routers from within the configuration of the global EIGRP AS 100.

On the customer routers, configure EIGRP AS 1 for the major network 172.16.0.0. Disable automatic summarization.

D1(config)# router eigrp 1

D1(config-router)# no auto-summary

D1(config-router)# network 10.1.11.0 0.0.0.255

D2(config)# router eigrp 1

D2(config-router)# no auto-summary

D2(config-router)# network 10.1.32.0 0.0.0.255

Given only the information above, will EIGRP form any adjacencies immediately?

Thus far, the only routers configured for EIGRP are the CE routers, which are isolated from each other. Therefore, EIGRP will not form any adjacencies.

On the PE routers, the configuration is more complex. Every IGP has a different method of configuring a VRF for it. To implement EIGRP for VRFs, start the EIGRP process by configuring EIGRP AS 100. Remember, this AS belongs to the provider and is not significant to the customer. If you were using EIGRP as the service provider’s IGP instead of OSPF, you would configure your network statements at this point .

R1(config)# router eigrp 100

R3(config)# router eigrp 100

Now, to configure EIGRP for an individual VRF instance, use the command address-family ipv4 vrf name, where name is the name of the VRF instance. Although each VPN must be logically separate from other IPv4 address spaces using VRF, this separation must extend not only to the routing table but to the routing protocols as well. The address-family command creates a logical segment of a routing protocol and its routes and adjacencies in order to separate it from other sets of routes and adjacencies. In this case, you will separate an EIGRP autonomous system from the EIGRP instance initiated with the router eigrp 100 command. Networks learned via this new autonomous system will be injected into the VRF routing table associated with the isolated EIGRP AS. It is also important to note that these networks will not be

advertised to any neighbors in EIGRP AS 100; it is completely separate from the rest of the EIGRP domain.

R1(config-router)# address-family ipv4 vrf customer autonomous-system 1

R1(config-router-af)# no auto-summary

R1(config-router-af)# network 10.1.11.0 0.0.0.255

R3(config-router)# address-family ipv4 vrf customer autonomous-system 1

R3(config-router-af)# no auto-summary

R3(config-router-af)# network 10.1.32.0 0.0.0.255

On the PE routers, display the default routing table with the show ip route command.. Display the VRF routing table with the show ip route vrf name command, where name is the VRF instance name.

R1#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

C        10.1.1.0/24 is directly connected, Loopback0

L        10.1.1.1/32 is directly connected, Loopback0

C        10.1.12.0/24 is directly connected, Ethernet0/0

L        10.1.12.1/32 is directly connected, Ethernet0/0

O        10.1.23.0/24 [110/20] via 10.1.12.2, 00:03:22, Ethernet0/0

O        10.2.2.1/32 [110/11] via 10.1.12.2, 00:03:22, Ethernet0/0

O        10.3.3.1/32 [110/21] via 10.1.12.2, 00:03:22, Ethernet0/0

R1#show ip route vrf customer

Routing Table: customer

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        10.1.11.0/24 is directly connected, Ethernet0/1

L        10.1.11.1/32 is directly connected, Ethernet0/1

198.51.100.0/25 is subnetted, 1 subnets

D        198.51.100.0 [90/409600] via 10.1.11.2, 00:02:22, Ethernet0/1

R3#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

O        10.1.1.1/32 [110/21] via 10.1.23.2, 00:33:49, Ethernet0/0

O        10.1.12.0/24 [110/20] via 10.1.23.2, 00:33:49, Ethernet0/0

C        10.1.23.0/24 is directly connected, Ethernet0/0

L        10.1.23.1/32 is directly connected, Ethernet0/0

O        10.2.2.1/32 [110/11] via 10.1.23.2, 00:33:49, Ethernet0/0

C        10.3.3.0/24 is directly connected, Loopback0

L        10.3.3.1/32 is directly connected, Loopback0

R3#show ip route vrf customer

Routing Table: customer

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        10.1.32.0/24 is directly connected, Ethernet0/1

L        10.1.32.1/32 is directly connected, Ethernet0/1

209.165.201.0/25 is subnetted, 1 subnets

D        209.165.201.0 [90/409600] via 10.1.32.2, 00:02:35, Ethernet0/1

The R1 and D1 routers do not possess routes to the customer networks on R3 and D2and vice versa. Explain why this occurs though EIGRP adjacencies have formed.

Since MP-BGP has not been configured to exchange VPNv4 routes between the two PE routers, no routes from the VRF on R1 will be advertised to R3 and vice versa. MP-BGP must be configured to advertise the networks and connect the two endpoints of the MPLS VPN.

Step 7: Configure BGP

Now that the PE routers are routing to the CE routers over VRF tables, you can set up the PE routers to exchange routes through BGP. First, configure BGP between R1 and R3 and have them peer between their loopback addresses. Synchronization should be disabled by default on newer IOS releases. If synchronization is not already disabled, then explicitly disable it using the no synchronization command.

R1(config)# router bgp 100

R1(config-router)# neighbor 10.3.3.1 remote-as 100

R1(config-router)# neighbor 10.3.3.1 update-source loopback0

R3(config)# router bgp 100

R3(config-router)# neighbor 10.1.1.1 remote-as 100

R3(config-router)# neighbor 10.1.1.1 update-source loopback0

To configure the exchange of VPNv4 routes over BGP, use the address-family vpnv4 command. At the address family prompt, activate the BGP neighbor for this address family with neighbor address activate command. Activating a neighbor for an address family allows BGP to send routes to and receive routes from the designated neighbor using the specified address family. By default, neighbors are only activated for IPv4.

The RTs are translated as extended BGP communities, so you must allow R1 and R3 to send both standard and extended communities over MP-BGP using the neighbor address send-community both command. The adjacencies may flap (temporarily go down and then come back up) when you activate the address family.

R1(config-router)# address-family vpnv4

R1(config-router-af)# neighbor 10.3.3.1 activate

R1(config-router-af)# neighbor 10.3.3.1 send-community both

R1(config-router-af)# exit

R3(config-router)# address-family vpnv4

R3(config-router-af)# neighbor 10.1.1.1 activate

R3(config-router-af)# neighbor 10.1.1.1 send-community both

R3(config-router-af)# exit

Finally, you need to configure BGP to redistribute the EIGRP routes in the VRF RIB into the BGP protocol so that these are advertised to the remote PE. Under the main BGP configuration prompt, enter another address family associated only with the routing table for the VRF customer. Redistribute the EIGRP routes that are associated with this VRF into BGP.

R1(config-router)# address-family ipv4 vrf customer

R1(config-router-af)# redistribute eigrp 1

R1(config-router-af)# exit

R1(config-router)# exit

R3(config-router)# address-family ipv4 vrf customer

R3(config-router-af)# redistribute eigrp 1

R3(config-router-af)# exit

R3(config-router)# exit

Based on the above configuration, will R1’s VRF RIB contain the 209.165.201.0/24 route that was originated by EIGRP on D2? Explain.

Yes, R1 will learn the 209.165.201.0/24  routes from R3 via BGP. R3 redistributes the route from EIGRP into BGP and propagates it to R1 via the MP-BGP adjacency that sends VPNv4 routes.

Will D1 learn the same routes via EIGRP? Explain.

No, D1 will not learn the same routes from R1 via EIGRP because no routes have yet been redistributed from BGP into EIGRP on R1.

Do you expect to see the redistributed routes as internal or external EIGRP routes on the CE routers? Explain.

Normally, when you redistribute into an EIGRP AS from another EIGRP AS or routing protocol, the routes are installed as external routes. You would expect these routes to be marked as ‘D EX’ in the output of the show ip route command. However, MPLS BGP solves this problem by checking the source and destination EIGRP AS. If the EIGRP AS numbers match, the route is installed in the EIGRP topology table as an internal route, and this feature persists into the VRF routing table (the RIB).

Enter the EIGRP instance that contains the VRF configuration on R1 and R3 and configure it to redistribute BGP routes. Since you are redistributing into EIGRP from BGP, the metrics are not comparable. Add a seed metric with a bandwidth of 64 kbps, 100 microseconds, reliability of 255/255, load of 1/255, and MTU of 1500 bytes.

R1(config)# router eigrp 100

R1(config-router)# address-family ipv4 vrf customer

R1(config-router-af)# redistribute bgp 100 metric 64 1000 255 1 1500

R3(config)# router eigrp 100

R3(config-router)# address-family ipv4 vrf customer

R3(config-router-af)# redistribute bgp 100 metric 64 1000 255 1 1500

Step 8: Investigate Control Plane Operation

Remember that MPLS differentiates the control plane from the forwarding plane. The control plane, represented by the routing table (the RIB) and the routing protocols, must operate so that the VRF routes reach remote PEs and are installed as necessary in the VRF routing tables. Not only the prefixes, but also the accompanying metrics and tags are important to the reconstruction of the route at the remote PE. Fortunately, MP-BGP allows you to send these metrics in the Network Layer Reachability Information (NLRI). Through this step and also Step 8, you will investigate the routing and forwarding information associated with the route to 209.165.201.0/24. Verify that the routes have propagated to the remote PE routers. Issue the show ip route vrf name command to see the VRF RIB. Notice the source of the routes on the PE routers.

R1#show ip route vrf customer

Routing Table: customer

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

C        10.1.11.0/24 is directly connected, Ethernet0/1

L        10.1.11.1/32 is directly connected, Ethernet0/1

B        10.1.32.0/24 [200/0] via 10.3.3.1, 00:08:18

198.51.100.0/25 is subnetted, 1 subnets

D        198.51.100.0 [90/409600] via 10.1.11.2, 00:23:43, Ethernet0/1

209.165.201.0/25 is subnetted, 1 subnets

B        209.165.201.0 [200/409600] via 10.3.3.1, 00:08:18

R3#show ip route vrf customer

Routing Table: customer

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

B        10.1.11.0/24 [200/0] via 10.1.1.1, 00:10:20

C        10.1.32.0/24 is directly connected, Ethernet0/1

L        10.1.32.1/32 is directly connected, Ethernet0/1

198.51.100.0/25 is subnetted, 1 subnets

B        198.51.100.0 [200/409600] via 10.1.1.1, 00:10:20

209.165.201.0/25 is subnetted, 1 subnets

D        209.165.201.0 [90/409600] via 10.1.32.2, 00:24:14, Ethernet0/1

You may be asking “Why does the source of the route to 209.165.201.0/24 on R1 point to 10.3.3.1 since that address would be routed based on the default routing table?” Consider that when an internally generated route is sent to an iBGP peer, BGP sets the NEXT-HOP attribute to be the advertising router. In this case, R3 generates the route in BGP by redistribution. The BGP peers are communicating between loopback interfaces, so the NEXT-HOP is set to the IP address of the BGP peer’s source interface. The VRF RIB thus points to an interface that must be reached through the default, global RIB. We will investigate the forwarding for packets destined for these networks in the next step.

On the CE routers, issue the show ip route command to see a full routing table.

D1#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

C        10.1.11.0/24 is directly connected, Ethernet2/0

L        10.1.11.2/32 is directly connected, Ethernet2/0

D        10.1.32.0/24 [90/307200] via 10.1.11.1, 00:08:16, Ethernet2/0

198.51.100.0/24 is variably subnetted, 2 subnets, 2 masks

C        198.51.100.0/25 is directly connected, Loopback0

L        198.51.100.1/32 is directly connected, Loopback0

209.165.201.0/25 is subnetted, 1 subnets

D        209.165.201.0 [90/435200] via 10.1.11.1, 00:08:16, Ethernet2/0

D2#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

D        10.1.11.0/24 [90/307200] via 10.1.32.1, 00:08:03, Ethernet2/0

C        10.1.32.0/24 is directly connected, Ethernet2/0

L        10.1.32.2/32 is directly connected, Ethernet2/0

198.51.100.0/25 is subnetted, 1 subnets

D        198.51.100.0 [90/435200] via 10.1.32.1, 00:08:03, Ethernet2/0

209.165.201.0/24 is variably subnetted, 2 subnets, 2 masks

C        209.165.201.0/25 is directly connected, Loopback0

L        209.165.201.1/32 is directly connected, Loopback0

On both the CE and PE routers, notice that the routes you redistributed from BGP into EIGRP are internal EIGRP routes because BGP preserves features of the EIGRP route while advertising the route to the other PEs. The PE encodes as much EIGRP information as possible into new extended communities TLV tuples (type, length, value) to preserve route characteristics through the VPN. This enables the remote PE router to reconstruct the EIGRP route with all of its characteristics, including the metric components, AS, TAG and, for external routes, the remote AS number, the remote ID, the remote protocol, and the remote metric. These are the EIGRP characteristics of a prefix that you can find in the topology table. If the EIGRP-advertised route is internal, the route is advertised as an internal route into the remote site if the destination AS matches the source AS carried by the BGP extended community attributes.

Display information on the VPNv4 BGP routes on R1 with the show bgp vpnv4 unicast all command.

R1#show bgp vpnv4 unicast all

BGP table version is 9, local router ID is 10.1.1.1

Status codes: s suppressed, d damped, h history, * valid, > best, i - internal,

r RIB-failure, S Stale, m multipath, b backup-path, f RT-Filter,

x best-external, a additional-path, c RIB-compressed,

Origin codes: i - IGP, e - EGP, ? - incomplete

RPKI validation codes: V valid, I invalid, N Not found

Network          Next Hop            Metric LocPrf Weight Path

Route Distinguisher: 100:1 (default for vrf customer)

*>  10.1.11.0/24     0.0.0.0                  0         32768 ?

*>i 10.1.32.0/24     10.3.3.1                 0    100      0 ?

*>  198.51.100.0/25  10.1.11.2           409600         32768 ?

*>i 209.165.201.0/25 10.3.3.1            409600    100      0 ?

R3#show bgp vpnv4 unicast all

BGP table version is 9, local router ID is 10.3.3.1

Status codes: s suppressed, d damped, h history, * valid, > best, i - internal,

r RIB-failure, S Stale, m multipath, b backup-path, f RT-Filter,

x best-external, a additional-path, c RIB-compressed,

Origin codes: i - IGP, e - EGP, ? - incomplete

RPKI validation codes: V valid, I invalid, N Not found

Network          Next Hop            Metric LocPrf Weight Path

Route Distinguisher: 100:1 (default for vrf customer)

*>i 10.1.11.0/24     10.1.1.1                 0    100      0 ?

*>  10.1.32.0/24     0.0.0.0                  0         32768 ?

*>i 198.51.100.0/25  10.1.1.1            409600    100      0 ?

*>  209.165.201.0/25 10.1.32.2           409600         32768 ?

Notice that the metric (MED value) in BGP is the metric advertised through EIGRP for that route as well. What does the value of the NEXT-HOP attribute for the 10.1.32.0/24 network on R3 indicate?

The value of 0.0.0.0 indicates that the route is originated by the local router.

What is the value of the BGP NEXT-HOP attribute for the 209.165.201.0/24 route on R1?

The next hop points to R3’s loopback interface, 10.3.3.1.

By which routing protocol and from which router was the route to 10.3.3.0/24 installed in the default routing table on R1?

R1 received the route to 10.3.3.0/24 from R2 via OSPF.

View more specific detail on a particular prefix using show bgp vpnv4 unicast all ip-address command. Notice that the MPLS label information is included. Execute this on both of the PEs. Remember that R3 is advertising the 209.165.201.0/24 prefix through BGP, while R1 is receiving the route through BGP NLRI.

R1# show bgp vpnv4 unicast all 209.165.201.0

BGP routing table entry for 100:1:209.165.201.0/24, version 15 Paths: (1 available, best #1, table customer)

Flag: 0x820

Not advertised to any peer Local

10.3.3.1 (metric 129) from 10.3.3.1 (10.3.3.1)

Origin incomplete, metric 2297856, localpref 100, valid, internal, best

Extended Community: RT:1:100

Cost:pre-bestpath:128:2297856 (default-2145185791) 0x8800:32768:0 0x8801:1:640000 0x8802:65281:1657856 0x8803:65281:1500

mpls labels in/out nolabel/20

R3# show bgp vpnv4 unicast all 209.165.201.1

BGP routing table entry for 100:1:209.165.201.0/24, version 15 Paths: (1 available, best #1, table customer)

Advertised to update-groups:

1 Local

10.1.32.1 from 0.0.0.0 (10.3.3.1)

Origin incomplete, metric 2297856, localpref 100, weight 32768, valid, sourced, best

Extended Community: RT:1:100

Cost:pre-bestpath:128:2297856 (default-2145185791) 0x8800:32768:0 0x8801:1:640000 0x8802:65281:1657856 0x8803:65281:1500

mpls labels in/out 20/nolabel

Notice that there are multiple values in the BGP extended communities. Recall that BGP sends the route information in NLRI as extended communities. These values are TLVs indicating such EIGRP attributes as the TAG, AS number, bandwidth, delay, reliability, load, MTU, and hop count.

Why is the origin code “incomplete”?

The route 209.165.201.1 was redistributed into BGP on R3. When routes are redistributed into BGP, they receive an origin code of incomplete by default.

What type of attribute carries the route target information in MP-BGP NLRI?

The route target is carried as an extended community value.

Notice the MPLS labels indicated for the BGP route above. The in-label of ‘nolabel’ on R1 indicates that R1 is not advertising a label for the prefix 209.165.201.0/24. The out-label of 21 is advertised by R3 and received by R1. This label is significant only on the path between R1 and R3. This label has been allocated by BGP on R3.

View the list of MPLS labels that are being used with BGP using show bgp vpnv4 unicast all labels.

| R1# show bgp vpnv4 | unicast all labels |  |
| --- | --- | --- |
| Network | Next Hop | In label/Out label |
| Route Distinguisher: 100:1 (customer) |  |  |
| 198.51.100.0/24 | 10.1.11.2 | 19/nolabel |
| 209.165.201.0/24 | 10.3.3.1 | nolabel/20 |
| 10.1.11.0/24 | 0.0.0.0 | 20/aggregate(customer) |
| 10.1.32.1.0/24 | 10.3.3.1 | nolabel/19 |
| R3# show bgp vpnv4 | unicast all labels |  |
| Network | Next Hop | In label/Out label |
| Route Distinguisher: 100:1 (customer) |  |  |
| 198.51.100.0/24 | 10.1.1.1 | nolabel/19 |
| 209.165.201.0/24 | 10.1.32.1.2 | 20/nolabel |
| 10.1.11.0/24 | 10.1.1.1 | nolabel/20 |
| 10.1.32.1.0/24 | 0.0.0.0 | 19/aggregate(customer) |

How has R1 learned the VPN label, label 20?

R1 has learned the label through BGP NLRI.

Will R1 or R2 learn the label via LDP?

No, R3 is not advertising the label via LDP.

Has the P router R3 learned about label 20 from R3? Explain.

No, the label will only be learned via BGP advertisement. Since BGP is not running on R2, R2 will not learn the label 20.

Finally, display the route attributes for the same prefix, 209.165.201.0/24, in the EIGRP topology table on R1 with the show ip eigrp vrf customer topology ip-prefix/mask command. Verify this against the originator of the EIGRP route in BGP, R3.

R1# show ip eigrp vrf customer topology 209.165.201.0/25

IP-EIGRP (AS 1): Topology entry for 209.165.201.0/25

State is Passive, Query origin flag is 1, 1 Successor(s), FD is 2297856 Routing Descriptor Blocks:

10.3.3.1, from VPNv4 Sourced, Send flag is 0x0

Composite metric is (2297856/0), Route is Internal (VPNv4 Sourced)

Vector metric:

Minimum bandwidth is 1544 Kbit

Total delay is 25000 microseconds

Reliability is 255/255

Load is 1/255

Minimum MTU is 1500

Hop count is 1

R3# show ip eigrp vrf customer topology 209.165.201.0/25

IP-EIGRP (AS 1): Topology entry for 209.165.201.0/25

State is Passive, Query origin flag is 1, 1 Successor(s), FD is 2297856 Routing Descriptor Blocks:

10.1.32.1.2 (E1/0), from 10.1.32.1.2, Send flag is 0x0

Composite metric is (2297856/128256), Route is Internal

Vector metric:

Minimum bandwidth is 1544 Kbit

Total delay is 25000 microseconds

Reliability is 255/255

Load is 1/255 Minimum

MTU is 1500

Hop count is 1

Notice that there is absolutely no difference in the EIGRP route parameters between R1 and R3. BGP encodes and decodes the information on the PE routers with no changes. Remember that R2 (a P router) has no knowledge of individual routes in the VRF tables on the PE routers. You can verify this with the show commands performed previously.

R2# show ip route

Codes: C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2 ia - IS-IS inter area, * - candidate default, U - per-user static route o - ODR, P - periodic downloaded static route

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 5 subnets, 2 masks

C	 10.0.12.0/24 is directly connected, E0/0

O 	10.3.3.1/32 [110/65] via 10.0.23.3, 1d00h, E0/1

C 	10.2.2.1/32 is directly connected, Loopback0

O 	10.1.1.1/32 [110/65] via 10.0.12.1, 1d00h, E0/0

C	 10.0.23.0/24 is directly connected, E0/1

R2# show ip route vrf customer

% IP routing table customer does not exist

Ping between the CE routers to verify connectivity through the MPLS VPN.

D1# ping 209.165.201.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.201.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 84/89/93 ms

D2# ping 198.51.100.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 198.51.100.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 84/86/88 ms

Step 9: Investigate Forwarding Plane Operation

Recall that MPLS has two tables, the Label Information Base (LIB) and the Label Forwarding Information Base (LFIB). Normally, LDP-allocated labels are advertised to LDP peers. BGP-allocated labels are advertised to BGP peers. BGP-allocated labels will be used by BGP peers as an MPLS label on packets destined for that network through the VPN. The BGP-allocated labels are only significant to the ingress and egress routers. P routers that are not BGP peers with the PE routers will not see the VPN label for the networks known by BGP.

In order to traverse the MPLS cloud, the packets need to be label-switched at every hop based on advertised labels. In order to ensure that VPN packets that reach the egress PE have the MPLS label needed to switch the packets once they arrive, the labels are stacked at the ingress PE. However, the packet still needs to be sent along the label-switched path.

Recall that the VRF RIB’s next hop for the networks known via the VPN point to the loopback on the egress PE. CEF uses the ‘inuse’ label for the BGP next hop as the outermost label for packets traveling through the MPLS VPN. First, however, CEF must push on the VPN label that will be used at the egress PE. Thus, CEF stacks the label in a sequential manner so that the VPN label is available at the egress PE, but the label to traverse the label-switched path through the P routers is pushed as the outermost label.

Take some time to study and understand the details of how this is possible. BGP, LDP, CEF, the LFIB, and the provider’s IGP are all involved in the use of MPLS labels as a VPN technology.

Once BGP learns the MPLS label to use as the VPN label, this information is entered into the CEF forwarding table on the ingress PE. Display the CEF forwarding entry for 209.165.201.0/24 on R1 with the show ip cef vrf name ip-address command.

R1# show ip cef vrf customer 209.165.201.0

209.165.201.0/25, version 12, epoch 0, cached adjacency to E0/0

packets, 0 bytes

tag information set

local tag: VPN-route-head

fast tag rewrite with E0/0, point2point, tags imposed: {16 20}

via 10.3.3.1, 0 dependencies, recursive

next hop 10.0.12.2, E0/0via 10.3.3.1/32

valid cached adjacency

tag rewrite with E0/0, point2point, tags imposed: {16 20}

CEF resolves the recursive lookup to the BGP next hop. Based on the labels learned by LDP, CEF may or may not apply the forwarding label to reach 10.3.3.0/24. In this case, LDP on R2 has advertised a forwarding label to R1. View the labels advertised to R1 via LDP using the show mpls ip binding command.

| R1# show mpls ip binding |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 10.1.1.1/32 |  | imp-null |  |  |  |
| in label: |  |  | lsr: 10.2.2.1:0 |  |  |
| out label: |  | 17 |  |  |  |
| 10.2.2.1/32 |  | 16 |  |  |  |
| in label: |  |  | lsr: 10.2.2.1:0 | inuse |  |
| out label: |  | imp-null |  |  |  |
| 10.3.3.1/32 |  | 17 |  |  |  |
| in label: |  |  |  |  |  |
|  | out label: | 16 | lsr: 10.2.2.1:0 | inuse |  |
| 10.0.12.0/24 |  | imp-null |  |  |  |
| in label: |  |  | lsr: 10.2.2.1:0 |  |  |
| out label: |  | imp-null |  |  |  |
| 010.0.23.0/24 |  | 18 |  |  |  |
| in label: |  |  | lsr: 10.2.2.1:0 | inuse |  |
| out label: |  | imp-null |  |  |  |

CEF pushes the label of 20 onto the packet first, then pushes the outer label of 16. The CEF forwarding table decides which path to use based of course on the default RIB. The route has been installed in the RIB by OSPF. Thus, the ingress PE imposes two labels in the sequence {16, 20} as shown in the CEF forwarding table above. Since the incoming VPN packets from R1 are encapsulated in MPLS frames, R2 acts according to the directives in its LFIB. R2 is also the penultimate hop in the label-switched path from R1 to R3’s loopback interface, and therefore pops the outermost label from the MPLS frame. Display the LFIB with the show mpls forwarding-table command.

| R2# show mpls forwarding-table |  |  |  | Bytes tag | Outgoing | Next Hop |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Local | Outgoing |  | Prefix |  |  |  |  |
| tag | tag or VC |  | or Tunnel Id | switched | interface |  |  |
| 16 | Pop | tag | 10.3.3.1/32 | 5175 | E0/1 | point2point |  |
| 17 | Pop | tag | 10.1.1.1/32 | 8079 | E0/0 | point2point |  |

Notice that the LFIB does not care whether there is an inner label or not, it simply performs the operation specified in the column labeled “Outgoing tag or VC.”

If you enable MPLS packet debugging on R2 using debug mpls packets then issue a ping from one CE to the other, you can see the MPLS packets being label-switched. The ICMP packets are forwarded inside MPLS frames through R2. Notice in the debug output that each ICMP echo request receives a reply which is label-switched on its return path through the MPLS network. When you are done, disable debugging.

| R2# debug mpls packets |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MPLS packet debugging |  |  | is on |  |  |  |  |
| D1# ping 209.165.201.1 |  |  |  |  |  |  |  |
| Type escape sequence to abort. |  |  |  |  | seconds: |  |  |
| Sending |  | 5, 100-byte ICMP Echos to 209.165.201.1, timeout is 2 |  |  |  |  |  |
| !!!!! |  | rate is 100 percent |  | (5/5), round-trip min/avg/max = | 84/87/92 ms |  |  |
| Success |  |  |  |  |  |  |  |
| R2# |  |  |  |  |  |  |  |
| *Feb | 3 | 20:55:57.422: MPLS: E0/0: recvd: CoS=0, TTL=254, |  |  | Label(s)=16/20 |  |  |
| *Feb | 3 | 20:55:57.422: MPLS: E0/1: xmit: CoS=0, TTL=253, Label(s)=20 |  |  |  |  |  |
| ! |  | 2 messages indicate |  | the label-switching of the ICMP | echo request |  |  |
| ! These |  |  |  |  |  |  |  |
| ! |  |  |  |  |  |  |  |
| *Feb | 3 | 20:55:57.478: MPLS: E0/1: recvd: CoS=0, TTL=254, |  |  | Label(s)=17/20 |  |  |
| *Feb | 3 | 20:55:57.478: MPLS: E0/0: xmit: CoS=0, TTL=253, Label(s)=20 |  |  |  |  |  |
| ! |  | 2 messages indicate |  | the label-switching of the ICMP | echo reply |  |  |
| ! These |  |  |  |  |  |  |  |
| ! | 3 | 20:55:57.510: MPLS: E0/0: recvd: CoS=0, TTL=254, |  |  | Label(s)=16/20 |  |  |
| *Feb |  |  |  |  |  |  |  |
| *Feb | 3 | 20:55:57.510: MPLS: E0/1: xmit: CoS=0, TTL=253, Label(s)=20 |  |  |  |  |  |
| *Feb | 3 | 20:55:57.566: MPLS: E0/1: recvd: CoS=0, TTL=254, |  |  | Label(s)=17/20 |  |  |
| *Feb | 3 | 20:55:57.566: MPLS: E0/0: xmit: CoS=0, TTL=253, Label(s)=20 |  |  |  |  |  |
| *Feb | 3 | 20:55:57.598: MPLS: E0/0: recvd: CoS=0, TTL=254, |  |  | Label(s)=16/20 |  |  |
| *Feb | 3 | 20:55:57.598: MPLS: E0/1: xmit: CoS=0, TTL=253, Label(s)=20 |  |  |  |  |  |
| *Feb | 3 | 20:55:57.654: MPLS: E0/1: recvd: CoS=0, TTL=254, |  |  | Label(s)=17/20 |  |  |
| *Feb | 3 | 20:55:57.654: MPLS: E0/0: xmit: CoS=0, TTL=253, Label(s)=20 |  |  |  |  |  |
| *Feb | 3 | 20:55:57.686: MPLS: E0/0: recvd: CoS=0, TTL=254, |  |  | Label(s)=16/20 |  |  |
| *Feb | 3 | 20:55:57.686: MPLS: E0/1: xmit: CoS=0, TTL=253, Label(s)=20 |  |  |  |  |  |
| *Feb | 3 | 20:55:57.742: MPLS: E0/1: recvd: CoS=0, TTL=254, |  |  | Label(s)=17/20 |  |  |
| *Feb | 3 | 20:55:57.742: MPLS: E0/0: xmit: CoS=0, TTL=253, Label(s)=20 |  |  |  |  |  |
| *Feb | 3 | 20:55:57.774: MPLS: E0/0: recvd: CoS=0, TTL=254, |  |  | Label(s)=16/20 |  |  |
| *Feb | 3 | 20:55:57.774: MPLS: E0/1: xmit: CoS=0, TTL=253, Label(s)=20 |  |  |  |  |  |

*Feb 3 20:55:57.830: MPLS: E0/1: recvd: CoS=0, TTL=254, Label(s)=17/20

*Feb 3 20:55:57.830: MPLS: E0/0: xmit: CoS=0, TTL=253, Label(s)=20

R2# undebug all

All possible debugging has been turned off

Continue tracing the label-switched path through the provider network to the egress PE, R3.Based on which forwarding table will the VPN packet be switched at R3? Explain.

The VPN packet will be forwarded based on the MPLS LFIB on R3 because it is encapsulated in an MPLS frame with a label of 20.

Display the MPLS LFIB on R3 using the show mpls forwarding-table command that you used on R2 previously.

| R3# show mpls forwarding-table |  |  | Bytes tag | Outgoing | Next Hop |  |
| --- | --- | --- | --- | --- | --- | --- |
| Local | Outgoing | Prefix |  |  |  |  |
| tag | tag or VC | or Tunnel Id | switched | interface | point2point |  |
| 16 | Pop tag | 10.0.12.0/24 | 0 | E0/1 |  |  |
| 17 | Pop tag | 10.2.2.1/32 | 0 | E0/1 | point2point |  |
| 18 | 17 | 10.1.1.1/32 | 0 | E0/1 | point2point |  |
| 19 | Aggregate | 10.1.32.1.0/24[V] | \ |  |  |  |
| 20 | Untagged | 209.165.201.0/24[V] | 2704 | Se0/1/0 | point2point |  |
|  |  |  | 2704 |  |  |  |

Notice that R3 forwards the decapsulated IP packet untagged to the Serial 0/1/0 egress interface because it was received with a label of 20. This is the label that BGP advertised to R1. R1’s CEF forwarding table encapsulated the IP packets within two MPLS labels {16 20} and then forwarded the packet to R2.

Conclusion

Issue the traceroute command from one CE to another to find that it is going through multiple Layer 3 hops. This is an important debugging tool because it can also be issued from a PE router with reference to a VRF.

D1# traceroute 209.165.201.1

Type escape sequence to abort.

Tracing the route to 209.165.201.1

| 1 | 10.1.11.1 0 msec 0 | msec 0 | msec |
| --- | --- | --- | --- |
| 2 | 10.1.12.2 126 msec 117 | msec 126 msec |  |
| 3 | 10.1.32.1. 59 msec | 50 msec | 50 msec |
| 4 | 10.1.32.2 50 msec 42 | msec * |  |

Fill in the following table tracing the path of packets from 10.1.11.2 to 209.165.201.1 in order to trace the packet’s path:

|  | Router |  | Incoming |  | Outgoing |  | Switched By |  |  | Incoming |  |  | Outgoing |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (MPLS/IP) |  | (MPLS/IP) |  | (CEF/LFIB) |  |  | Label(s) |  |  | Label(s) |  |  |
|  | D1 |  | — |  | IP |  | CEF |  |  | — |  |  | — |  |  |
|  | R1 |  | IP |  | MPLS |  | CEF |  |  | — |  | {16 20} |  |  |  |
|  | R2 |  | MPLS |  | MPLS |  | LFIB |  | {16 20} |  |  | {20} |  |  |  |
|  | R3 |  | MPLS |  | IP |  | LFIB |  | {20} |  |  |  | — |  |  |
|  | D2 |  | IP |  | — |  | LFIB |  |  | — |  |  | — |  |  |

Given the output shown below on each of the routers, trace the return path from 209.165.201.1 to 10.1.11.2 by filling in the chart.

D2# show ip cef 10.1.11.2

10.1.11.0/24, version 22, epoch 0, cached adjacency to E2/0

0 packets, 0 bytes

via 10.1.32.1.3, E0/0, 0 dependencies

next hop 10.1.32.1.3, E0/0

valid cached adjacency

R3# show ip cef vrf customer 10.1.11.2

10.1.11.0/24, version 6, epoch 0, cached adjacency to E0/0

| 0 packets, |  | 0 bytes |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| tag information set |  |  |  |  |  |  |  |
| local tag: VPN-route-head |  |  |  |  | tags imposed: {17 20} |  |  |
| fast tag rewrite with E0/1, point2point, |  |  |  |  |  |  |  |
| via 10.1.1.1, 0 dependencies, recursive |  |  |  |  |  |  |  |
| next hop 10.0.23.2, E0/0 via 10.1.1.1/32 |  |  |  |  |  |  |  |
| valid cached adjacency |  |  |  |  | imposed: {17 20} |  |  |
| tag rewrite with E0/1, point2point, tags |  |  |  |  |  |  |  |
| R2# show mpls forwarding-table |  |  |  | Bytes tag | Outgoing | Next Hop |  |
| Local | Outgoing |  | Prefix |  |  |  |  |
| tag | tag | or VC | or Tunnel Id | switched | interface | point2point |  |
| 16 | Pop | tag | 10.3.3.1/32 | 15601 | E0/1 |  |  |
| 17 | Pop | tag | 10.1.1.1/32 | 25413 | E0/0 | point2point |  |
| R1# show mpls forwarding-table |  |  |  | Bytes tag | Outgoing | Next Hop |  |
| Local | Outgoing |  | Prefix |  |  |  |  |
| tag | tag | or VC | or Tunnel Id | switched | interface | point2point |  |
| 16 | Pop | tag | 10.2.2.1/32 | 0 | E0/0 |  |  |
| 17 | 16 | tag | 10.3.3.1/32 | 0 | E0/0 | point2point |  |
| 18 | Pop |  | 10.0.23.0/24 | 0 | E0/0 | point2point |  |
| 19 Untagged 198.51.100.0/24(V) 0 e0/1 |  |  |  |  |  | 10.1.11.2 |  |

20	       Aggregate          10.1.11.0/24(V)  \0

|  | Router |  | Incoming |  | Outgoing |  | Switched By |  |  | Incoming |  |  | Outgoing |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (MPLS/IP) |  | (MPLS/IP) |  | (CEF/LFIB) |  |  | Label(s) |  |  | Label(s) |  |  |
|  | D2 |  | — |  | IP |  | CEF |  |  | — |  |  | — |  |  |
|  | R3 |  | IP |  | MPLS |  | CEF |  |  | — |  | {17 20} |  |  |  |
|  | R2 |  | MPLS |  | MPLS |  | LFIB |  | {17 20} |  |  | {20} |  |  |  |
|  | R1 |  | MPLS |  | IP |  | LFIB |  | {20} |  |  |  | — |  |  |
|  | D1 |  | IP |  | — |  | LFIB |  |  | — |  |  | — |  |  |

R1

R1#

R1#sh run

Building configuration...

Current configuration : 2114 bytes

!

! Last configuration change at 14:01:02 UTC Tue Apr 25 2023

!

version 15.5

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname R1

!

boot-start-marker

boot-end-marker

!

!

!

no aaa new-model

!

!

!

bsd-client server url https://cloudsso.cisco.com/as/token.oauth2

mmi polling-interval 60

no mmi auto-configure

no mmi pvc

mmi snmp-timeout 180

!

!

!

!

!

!

!

!

!

!

!

!

!

!

ip vrf customer

rd 100:1

route-target export 1:100

route-target import 1:100

!

!

!

!

no ip domain lookup

ip cef

no ipv6 cef

!

multilink bundle-name authenticated

!

!

!

!

!

!

!

cts logging verbose

!

!

!

redundancy

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

interface Loopback0

ip address 10.1.1.1 255.255.255.255

!

interface Ethernet0/0

ip address 10.1.12.1 255.255.255.0

mpls ip

!

interface Ethernet0/1

ip vrf forwarding customer

ip address 10.1.11.1 255.255.255.0

!

interface Ethernet0/2

no ip address

shutdown

!

interface Ethernet0/3

no ip address

shutdown

!

interface Serial1/0

no ip address

shutdown

serial restart-delay 0

!

interface Serial1/1

no ip address

shutdown

serial restart-delay 0

!

interface Serial1/2

no ip address

shutdown

serial restart-delay 0

!

interface Serial1/3

no ip address

shutdown

serial restart-delay 0

!

!

router eigrp 100

!

address-family ipv4 vrf customer autonomous-system 1

redistribute bgp 100 metric 64 1000 255 1 1500

network 10.1.11.0 0.0.0.255

exit-address-family

!

router ospf 1

network 10.1.1.1 0.0.0.0 area 0

network 10.1.12.0 0.0.0.255 area 0

!

router bgp 100

bgp log-neighbor-changes

neighbor 10.3.3.1 remote-as 100

neighbor 10.3.3.1 update-source Loopback0

!

address-family vpnv4

neighbor 10.3.3.1 activate

neighbor 10.3.3.1 send-community both

exit-address-family

!

address-family ipv4 vrf customer

redistribute eigrp 1

exit-address-family

!

ip forward-protocol nd

!

!

no ip http server

no ip http secure-server

!

!

!

mpls ldp router-id Loopback0 force

!

control-plane

!

!

!

!

!

!

!

!

line con 0

exec-timeout 0 0

logging synchronous

line aux 0

line vty 0 4

login

transport input none

!

!

End

R2

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

interface Loopback0

ip address 10.2.2.1 255.255.255.255

!

interface Ethernet0/0

ip address 10.1.12.2 255.255.255.0

mpls ip

!

interface Ethernet0/1

ip address 10.1.23.2 255.255.255.0

mpls ip

!

interface Ethernet0/2

no ip address

shutdown

!

interface Ethernet0/3

ip address 192.168.179.1 255.255.255.0

shutdown

!

interface Serial1/0

no ip address

shutdown

serial restart-delay 0

!

interface Serial1/1

no ip address

shutdown

serial restart-delay 0

!

interface Serial1/2

no ip address

shutdown

serial restart-delay 0

!

interface Serial1/3

no ip address

shutdown

serial restart-delay 0

!

router ospf 1

network 10.0.0.0 0.255.255.255 area 0

!

ip forward-protocol nd

!

!

no ip http server

no ip http secure-server

!

!

!

mpls ldp router-id Loopback0 force

!

control-plane

!

!

!

!

!

!

!

!

line con 0

exec-timeout 0 0

logging synchronous

line aux 0

line vty 0 4

login

transport input none

!

ntp server pnpntpserver.localdomain

!

End

R3

R3#

R3#sh run

Building configuration...

Current configuration : 2114 bytes

!

! Last configuration change at 13:50:51 UTC Tue Apr 25 2023

!

version 15.5

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname R3

!

boot-start-marker

boot-end-marker

!

!

!

no aaa new-model

!

!

!

bsd-client server url https://cloudsso.cisco.com/as/token.oauth2

mmi polling-interval 60

no mmi auto-configure

no mmi pvc

mmi snmp-timeout 180

!

!

!

!

!

!

!

!

!

!

!

!

!

!

ip vrf customer

rd 100:1

route-target export 1:100

route-target import 1:100

!

!

!

!

no ip domain lookup

ip cef

no ipv6 cef

!

multilink bundle-name authenticated

!

!

!

!

!

!

!

cts logging verbose

!

!

!

redundancy

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

interface Loopback0

ip address 10.3.3.1 255.255.255.255

!

interface Ethernet0/0

ip address 10.1.23.1 255.255.255.0

mpls ip

!

interface Ethernet0/1

ip vrf forwarding customer

ip address 10.1.32.1 255.255.255.0

!

interface Ethernet0/2

no ip address

shutdown

!

interface Ethernet0/3

no ip address

shutdown

!

interface Serial1/0

no ip address

shutdown

serial restart-delay 0

!

interface Serial1/1

no ip address

shutdown

serial restart-delay 0

!

interface Serial1/2

no ip address

shutdown

serial restart-delay 0

!

interface Serial1/3

no ip address

shutdown

serial restart-delay 0

!

!

router eigrp 100

!

address-family ipv4 vrf customer autonomous-system 1

redistribute bgp 100 metric 64 1000 255 1 1500

network 10.1.32.0 0.0.0.255

exit-address-family

!

router ospf 1

network 10.1.23.0 0.0.0.255 area 0

network 10.3.3.1 0.0.0.0 area 0

!

router bgp 100

bgp log-neighbor-changes

neighbor 10.1.1.1 remote-as 100

neighbor 10.1.1.1 update-source Loopback0

!

address-family vpnv4

neighbor 10.1.1.1 activate

neighbor 10.1.1.1 send-community both

exit-address-family

!

address-family ipv4 vrf customer

redistribute eigrp 1

exit-address-family

!

ip forward-protocol nd

!

!

no ip http server

no ip http secure-server

!

!

!

mpls ldp router-id Loopback0 force

!

control-plane

!

!

!

!

!

!

!

!

line con 0

exec-timeout 0 0

logging synchronous

line aux 0

line vty 0 4

login

transport input none

!

!

end

D1

D1#

D1#sh run

Building configuration...

Current configuration : 1300 bytes

!

! Last configuration change at 13:14:20 UTC Tue Apr 25 2023

!

version 15.2

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

service compress-config

!

hostname D1

!

boot-start-marker

boot-end-marker

!

!

!

no aaa new-model

!

!

!

!

!

!

!

!

no ip domain-lookup

ip cef

no ipv6 cef

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

interface Loopback0

ip address 198.51.100.1 255.255.255.128

!

interface Ethernet0/0

shutdown

!

interface Ethernet0/1

shutdown

!

interface Ethernet0/2

shutdown

!

interface Ethernet0/3

shutdown

!

interface Ethernet1/0

shutdown

!

interface Ethernet1/1

shutdown

!

interface Ethernet1/2

shutdown

!

interface Ethernet1/3

shutdown

!

interface Ethernet2/0

no switchport

ip address 10.1.11.2 255.255.255.0

!

interface Ethernet2/1

shutdown

!

interface Ethernet2/2

shutdown

!

interface Ethernet2/3

shutdown

!

!

router eigrp 1

network 10.0.0.0

network 198.51.100.0 0.0.0.127

!

ip forward-protocol nd

!

ip http server

!

ip ssh server algorithm encryption aes128-ctr aes192-ctr aes256-ctr

ip ssh client algorithm encryption aes128-ctr aes192-ctr aes256-ctr

!

!

!

!

!

control-plane

!

!

line con 0

exec-timeout 0 0

logging synchronous

line aux 0

line vty 0 4

!

!

!

End

D2

D2#sh run

Building configuration...

Current configuration : 1313 bytes

!

! Last configuration change at 14:08:15 UTC Tue Apr 25 2023

!

version 15.2

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

service compress-config

!

hostname D2

!

boot-start-marker

boot-end-marker

!

!

!

no aaa new-model

!

!

!

!

!

!

!

!

no ip domain-lookup

ip cef

no ipv6 cef

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

interface Loopback0

ip address 209.165.201.1 255.255.255.128

!

interface Ethernet0/0

shutdown

!

interface Ethernet0/1

shutdown

!

interface Ethernet0/2

shutdown

!

interface Ethernet0/3

shutdown

!

interface Ethernet1/0

shutdown

!

interface Ethernet1/1

shutdown

!

interface Ethernet1/2

shutdown

!

interface Ethernet1/3

shutdown

!

interface Ethernet2/0

no switchport

ip address 10.1.32.2 255.255.255.0

!

interface Ethernet2/1

shutdown

!

interface Ethernet2/2

shutdown

!

interface Ethernet2/3

shutdown

!

!

router eigrp 1

network 10.1.32.0 0.0.0.255

network 209.165.201.0 0.0.0.127

!

ip forward-protocol nd

!

ip http server

!

ip ssh server algorithm encryption aes128-ctr aes192-ctr aes256-ctr

ip ssh client algorithm encryption aes128-ctr aes192-ctr aes256-ctr

!

!

!

!

!

control-plane

!

!

line con 0

exec-timeout 0 0

logging synchronous

line aux 0

line vty 0 4

!

!

!

end