# Lab - Implement VRF-Lite (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Topology

# Addressing Table

| Device | Interface | IPv4 Address |
| --- | --- | --- |
| R1 | E0/0 | 10.1.2.1/24 |
| R1 | E0/1.5 | 10.1.2.1/24 |
| R1 | E0/1.8 | 10.1.3.1/24 |
| R1 | S1/0 | 10.1.3.1/25 |
| R2 | E0/0 | 10.2.3.2/24 |
| R2 | Loopback0 | 192.168.2.1/24 |
| R3 | S1/0 | 10.1.3.3/25 |
| R3 | Loopback0 | 192.168.3.1/27 |
| D1 | E1/0 | 10.1.2.2/24 |
| D1 | VLAN 11 | 192.168.2.1/24 |
| D2 | E1/0 | 10.1.3.2/24 |
| D2 | VLAN 11 | 192.168.3.1/24 |

# Objectives

Part 1: Build the Network and Configure Basic Device Settings

Part 2: Configure and Verify VRF and Interface Addressing

Part 3: Configure and Verify Static Routing for Reachability Inside Each VRF

# Background / Scenario

By default, all interfaces on a router are included in the global routing table. Service providers must be able to virtualize the router, thus creating multiple, virtual routing tables. Virtual Routing and Forwarding (VRF) can do just that. VRF-Lite is VRF without the MPLS component.

In this lab, you will work on R1, playing the part of a service provider router, as it supports two customers who have the same addressing scheme configured. Your task is to deploy VRF-Lite and static routing so that the customers have full reachability within their network.

Note: This lab is an exercise in developing, deploying, and verifying VRF-Lite, and does not reflect networking best practices.

Note: The routers and switches used with CCNP hands-on labs are Cisco 4221 and Cisco 3650, both with Cisco IOS XE Release 16.9.4 (universalk9 image), and Cisco 2960+ with IOS release 15.2 (lanbase image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs

Note: Ensure that the routers and switches have been erased and have no startup configurations. If you are unsure contact your instructor.

Note: The PCs used in this lab do not require addressing. They are needed to bring interface VLAN 11 up.

Instructor Note: Refer to the Instructor Lab Manual for the procedures to initialize and reload devices.

# Required Resources

3 Routers (Cisco 4221 with Cisco IOS XE Release 16.9.4 universal image or comparable)

2 Switches (Cisco 3650 with Cisco IOS XE release 16.9.4 universal image or comparable)

1 Switch (Cisco 2960+ with Cisco IOS release 15.2 lanbase image or comparable)

2 PCs (Windows with a terminal emulation program, such as Tera Term)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet and serial cables as shown in the topology

## Build the Network and Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings on all devices.

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

### Configure basic settings for each device.

Console into each device, enter global configuration mode, and apply the basic settings. A command list for each device using the following startup configurations.

Open configuration window

Router R1

enable

configure terminal

hostname R1

no ip domain lookup

banner motd # R1, Implement VRF-Lite #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

Router R2

enable

configure terminal

hostname R2

no ip domain lookup

banner motd # R2, Implement VRF-Lite #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

interface e0/0

ip address 10.1.2.2 255.255.255.0

no shutdown

exit

interface loopback 0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

ip route 0.0.0.0 0.0.0.0 e0/0 10.1.2.1

Router R3

enable

configure terminal

hostname R3

no ip domain lookup

banner motd # R3, Implement VRF-Lite #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

interface S1/0

ip address 10.1.3.2 255.255.255.0

no shutdown

exit

interface loopback 0

ip address 192.168.3.1 255.255.255.0

no shutdown

exit

ip route 0.0.0.0 0.0.0.0 S1/0 10.1.3.1

Switch D1

enable

configure terminal

hostname D1

no ip domain lookup

ip routing

banner motd # D1, Implement VRF-Lite #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

interface range e0/0-3 , e1/0-3

shutdown

exit

interface e1/0

no switchport

ip address 10.1.2.2 255.255.255.0

no shutdown

exit

vlan 11

name LOCAL_VLAN

exit

interface vlan 11

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface e1/2

switchport mode access

switchport access vlan 11

no shutdown

exit

ip route 0.0.0.0 0.0.0.0 e1/0 10.1.2.1

Switch D2

enable

configure terminal

hostname D2

no ip domain lookup

ip routing

banner motd # D2, Implement VRF-Lite #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

interface range e1/0-3, e0/0

shutdown

exit

interface e1/0

no switchport

ip address 10.1.3.2 255.255.255.0

no shutdown

exit

vlan 11

name LOCAL_VLAN

exit

interface vlan 11

ip address 192.168.3.1 255.255.255.0

no shutdown

exit

interface e1/2

switchport mode access

switchport access vlan 11

no shutdown

exit

ip route 0.0.0.0 0.0.0.0 e1/0 10.1.3.1

Switch A1

enable

configure terminal

hostname A1

no ip domain lookup

banner motd # A1, Implement VRF-Lite #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

interface range e0/0-3 , e1/0-3

shutdown

exit

vlan 5

name D1

exit

vlan 8

name D2

exit

interface e1/1

switchport trunk encapsulation dot1q

switchport mode trunk

switchport nonegotiate

no shutdown

exit

interface e0/0

switchport mode access

switchport access vlan 5

no shutdown

exit

interface e0/2

switchport mode access

switchport access vlan 8

no shutdown

Set the clock on each router to UTC time.

Save the running configuration to startup-config.

Close configuration window

## Configure and Verify VRF and Interface Addressing

In Part 2, you will configure and verify VRF-Lite on R1. The other devices, R2, R3, D1, D2, and A1 require no additional configuration. Once again, the configuration being used here is not meant to represent best practice, but to assess your ability to complete the required configurations.

### On R1, create the required VRFs.

Create the Customer_A and Customer_B VRFs, and initialize them for both IPv4. The VRF names are case sensitive.

Open configuration window

R1(config)# vrf definition Customer_A

R1(config-vrf)# address-family ipv4

R1(config-vrf-af)# exit

R1(config-vrf)# vrf definition Customer_B

R1(config-vrf)# address-family ipv4

R1(config-vrf-af)# exit

Configure interfaces E0/0 and S1/0 for the Customer_A network.

R1(config)# interface e0/0

R1(config-if)# vrf forwarding Customer_A

R1(config-if)# ip address 10.1.2.1 255.255.255.0

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface S1/0

R1(config-if)# vrf forwarding Customer_A

R1(config-if)# ip address 10.1.3.1 255.255.255.0

R1(config-if)# no shutdown

R1(config-if)# exit

Configure R1 interface E0/1 to support the Customer_B networks. E0/1 will be performing inter-VLAN routing between VLANs 5 and 8.

R1(config)# interface e0/1

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface e0/1.5

R1(config-subif)# encapsulation dot1q 5

R1(config-subif)# vrf forwarding Customer_B

R1(config-subif)# ip address 10.1.2.1 255.255.255.0

R1(config-subif)# exit

R1(config)# interface e0/1.8

R1(config-subif)# encapsulation dot1q 8

R1(config-subif)# vrf forwarding Customer_B

R1(config-subif)# ip address 10.1.3.1 255.255.255.0

R1(config-subif)# end

### Verify the VRF-Lite configuration.

Verify the interface assignments using the show ip vrf interfaces command.

R1# show ip vrf interfaces

Interface              IP-Address      VRF                              Protocol

Gi0/0/0                10.1.2.1        Customer_A                       up

Se0/1/0                10.1.3.1        Customer_A                       up

Gi0/0/1.5              10.1.2.1        Customer_B                       up

Gi0/0/1.8              10.1.3.1        Customer_B                       up

Verify the VRF routing tables with the show ip route vrf vrf_name command.

R1# show ip route vrf Customer_A | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 4 subnets, 2 masks

C        10.1.2.0/24 is directly connected, Ethernet0/0

L        10.1.2.1/32 is directly connected, Ethernet0/0

C        10.1.3.0/24 is directly connected, Serial1/0

L        10.1.3.1/32 is directly connected, Serial1/0

Verify next-hop reachability within each vrf with the ping vrf vrf_name address command.

R1# ping vrf Customer_A 10.1.2.2

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.1.2.2, timeout is 2 seconds:

.!!!!

Success rate is 80 percent (4/5), round-trip min/avg/max = 1/1/1 ms

R1#  ping vrf Customer_A 10.1.3.2

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.1.3.2, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 2/2/3 ms

## Configure and Verify Static Routing for Reachability Inside Each VRF

In Part 3, you will configure static routing so that all networks are reachable within their respective VRFs. At the end of this part, R1 should be able to successfully source a ping from interface loopback0 to R3 interface loopback0, and D1 should be able to successfully source a ping from interface VLAN 11 to D2 interface VLAN 11. Once again, the way these networks are being implemented is not meant to represent best practice, but to assess your ability to complete the required configurations.

### Verify that distant networks are not reachable within each VRF.

In this step, you will check to make sure that distant networks are not reachable from R1 within each VRF.

On R1, issue the commands ping vrf Customer_A 192.168.2.1 and ping vrf Customer_A 192.168.3.1. Neither should succeed.

Open configuration window

R1# ping vrf Customer_A 192.168.2.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.2.1, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

R1# ping vrf Customer_A 192.168.3.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.3.1, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

On R1, issue the commands ping vrf Customer_B 192.168.2.1 and ping vrf Customer_B 192.168.3.1. Neither should succeed.

R1# ping vrf Customer_B 192.168.2.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.2.1, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

R1# ping vrf Customer_B 192.168.3.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.3.1, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

### Configure static routing at R1 for each VRF.

In this step, you will configure R1 so that it can reach distant networks in each VRF. The neighbor systems (D1, D2, R2, and R3) have static routes already configured, so as soon as you correctly install these static routes, there will be full reachability within each VRF.

On R1, create static routes for the distant networks in the Customer_A VRF using the ip route vrf vrf_name destination_network next-hop command.

R1(config)# ip route vrf Customer_A 192.168.2.0 255.255.255.0 e0/0 10.1.2.2

R1(config)# ip route vrf Customer_A 192.168.3.0 255.255.255.0 S1/0 10.1.3.2

Use the example above to correctly configure fully specified static routes for the Customer_B network.

R1(config)# ip route vrf Customer_B 192.168.2.0 255.255.255.0 Ethernet0/1.5 10.1.2.2

R1(config)# ip route vrf Customer_B 192.168.3.0 255.255.255.0 Ethernet0/1.8 10.1.3.2

### Verify full reachability within each VRF.

On R2, ping the IPv4 addresses of R3 interface Loopback0 using a source address of R2 interface Loopback0. All pings should be successful.

R2# ping 192.168.3.1 source loopback0

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.3.1, timeout is 2 seconds:

Packet sent with a source address of 192.168.2.1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 2/2/3 ms

On D1, ping the IPv4 addresses of D2 interface VLAN 11 using a source address of D1 interface VLAN 11. All pings should be successful.

D1# ping 192.168.3.1 source vlan11

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.3.1, timeout is 2 seconds:

Packet sent with a source address of 192.168.2.1

!!!!!