# Lab - Implement BGP Path Manipulation (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Topology

# Addressing Table

| Device | Interface | IPv4 Address |
| --- | --- | --- |
| R1 R1 R1 R1 R1 | E0/0 | 10.1.2.1/24 |
|  | S1/0 | 10.1.3.1/25 |
|  | S1/1 | 10.1.3.129/25 |
|  | Loopback0 | 192.168.1.1/27 |
|  | Loopback1 | 192.168.1.65/26 |
| R2 R2 R2 R2 | E0/0 | 10.1.2.2/24 |
|  | E0/1 | 10.2.3.2/24 |
|  | Loopback0 | 192.168.2.1/27 |
|  | Loopback1 | 192.168.2.65/26 |
| R3 R3 R3 R3 R3 | E0/0 | 10.2.3.3/24 |
|  | S1/0 | 10.1.3.3/25 |
|  | S1/1 | 10.1.3.130/25 |
|  | Loopback0 | 192.168.3.1/27 |
|  | Loopback1 | 192.168.3.65/26 |

# Objectives

Part 1: Build the Network and Configure Basic Device Settings and Interface Addressing

Part 2: Configure and Verify Multi-Protocol BGP on all Routers

Part 3: Configure and Verify BGP Path Manipulation Settings on all Routers

# Background / Scenario

The default settings in BGP allow for a great deal of undesired route information to pass between autonomous systems. In this lab you will configure Multi-Protocol BGP and implement various path manipulation options for both IPv4.

Note: This lab is an exercise in developing, deploying, and verifying various path manipulation tools for BGP, and does not reflect networking best practices.

Note: The routers used with CCNP hands-on labs are Cisco 4221 with Cisco IOS XE Release 16.9.4 (universalk9 image). Other routers and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs.

Note: Ensure that the routers have been erased and have no startup configurations. If you are unsure contact your instructor.

Instructor Note: Refer to the Instructor Lab Manual for the procedures to initialize and reload devices.

# Required Resources

3 Routers (Cisco 4221 with Cisco IOS XE Release 16.9.4 universal image or comparable)

1 PC (Choice of operating system with a terminal emulation program installed)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet and serial cables as shown in the topology

# Instructions

## Build the Network and Configure Basic Device Settings and Interface Addressing

In Part 1, you will set up the network topology and configure basic settings and interface addressing on routers.

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

### Configure basic settings for each router.

Console into each router, enter global configuration mode, and apply the basic settings and interface addressing. A command list for each router is listed below to perform initial configuration.

Open configuration window

Router R1

no ip domain lookup

hostname R1

line con 0

exec-timeout 0 0

logging synchronous

banner motd # This is R1, BGP Path Manipulation Lab #

interface e0/0

ip address 10.1.2.1 255.255.255.0

no shutdown

interface S1/0

ip address 10.1.3.1 255.255.255.128

no shutdown

interface S1/1

ip address 10.1.3.129 255.255.255.128

no shutdown

interface loopback 0

ip address 192.168.1.1 255.255.255.224

no shutdown

interface loopback 1

ip address 192.168.1.65 255.255.255.192

no shutdown

Router R2

no ip domain lookup

hostname R2

line con 0

exec-timeout 0 0

logging synchronous

banner motd # This is R2, BGP Path Manipulation Lab #

interface e0/0

ip address 10.1.2.2 255.255.255.0

no shutdown

interface e0/1

ip address 10.2.3.2 255.255.255.0

no shutdown

interface loopback 0

ip address 192.168.2.1 255.255.255.224

no shutdown

interface loopback 1

ip address 192.168.2.65 255.255.255.192

no shutdown

Router R3

no ip domain lookup

hostname R3

line con 0

exec-timeout 0 0

logging synchronous

banner motd # This is R3, BGP Path Manipulation Lab #

interface e0/0

ip address 10.2.3.3 255.255.255.0

no shutdown

interface S1/0

ip address 10.1.3.3 255.255.255.128

no shutdown

interface S1/1

ip address 10.1.3.130 255.255.255.128

no shutdown

interface loopback 0

ip address 192.168.3.1 255.255.255.224

no shutdown

interface loopback 1

ip address 192.168.3.65 255.255.255.192

no shutdown

Save the running configuration to startup-config.

Close configuration window

## Configure and Verify Multi-Protocol BGP on all Routers

In Part 2, you will configure and verify Multi-Protocol BGP on all routers to achieve full connectivity between the routers. The text below provides you with the complete configuration for R1. You will use this to inform your configuration of R2 and R3. The configuration being used here is not meant to represent best practice, but to assess your ability to complete the required configurations.

### On R1, create the core BGP configuration.

Enter BGP configuration mode from global configuration mode, specifying AS 6500.

Open configuration window

R1(config)# router bgp 6500

Configure the BGP router-id for R1.

R1(config-router)# bgp router-id 1.1.1.1

Disable the default IPv4 unicast address family behavior.

R1(config-router)# no bgp default ipv4-unicast

Based on the topology diagram, configure all the designated neighbors for R1.

R1(config-router)# neighbor 10.1.2.2 remote-as 500

R1(config-router)# neighbor 10.1.3.3 remote-as 300

R1(config-router)# neighbor 10.1.3.130 remote-as 300

### On R1, configure the IPv4 unicast address family.

Enter the IPv4 unicast address family configuration mode.

R1(config-router)# address-family ipv4 unicast

Configure network statements for the IPv4 networks attached to interfaces loopback0 and loopback1. Remember that BGP does not work the same way that an IGP does, and that the network statement has no impact on neighbor adjacency; it is used solely for advertising purposes.

R1(config-router-af)# network 192.168.1.0 mask 255.255.255.224

R1(config-router-af)# network 192.168.1.64 mask 255.255.255.192

activate the IPv4 neighbors.

R1(config-router-af)# neighbor 10.1.2.2 activate

R1(config-router-af)# neighbor 10.1.3.3 activate

R1(config-router-af)# neighbor 10.1.3.130 activate

### Configure MP-BGP on R2 and R3 as you did in the previous step.

R2(config)# router bgp 500

R2(config-router)# bgp router-id 2.2.2.2

R2(config-router)# no bgp default ipv4-unicast

R2(config-router)# neighbor 10.1.2.1 remote-as 6500

R2(config-router)# neighbor 10.2.3.3 remote-as 300

R2(config-router)# address-family ipv4

R2(config-router-af)# network 192.168.2.0 mask 255.255.255.224

R2(config-router-af)# network 192.168.2.64 mask 255.255.255.192

R2(config-router-af)# neighbor 10.1.2.1 activate

R2(config-router-af)# neighbor 10.2.3.3 activate

R2(config-router-af)# exit-address-family

R3(config)# router bgp 300

R3(config-router)# bgp router-id 3.3.3.3

R3(config-router)# no bgp default ipv4-unicast

R3(config-router)# neighbor 10.1.3.1 remote-as 6500

R3(config-router)# neighbor 10.1.3.129 remote-as 6500

R3(config-router)# neighbor 10.2.3.2 remote-as 500

R3(config-router)# address-family ipv4

R3(config-router-af)# network 192.168.3.0 mask 255.255.255.224

R3(config-router-af)# network 192.168.3.64 mask 255.255.255.192

R3(config-router-af)# neighbor 10.1.3.1 activate

R3(config-router-af)# neighbor 10.1.3.129 activate

R3(config-router-af)# neighbor 10.2.3.2 activate

R3(config-router-af)# exit-address-family

### Verify that MP-BGP is operational.

Use the show bgp ipv4 unicast summary commands to verify that BGP has established three IPv4 adjacencies and received four prefixes from each neighbor.

R1# show bgp ipv4 unicast summary

BGP router identifier 1.1.1.1, local AS number 6500

BGP table version is 9, main routing table version 9

6 network entries using 1488 bytes of memory

14 path entries using 1904 bytes of memory

5/3 BGP path/bestpath attribute entries using 1400 bytes of memory

4 BGP AS-PATH entries using 128 bytes of memory

0 BGP route-map cache entries using 0 bytes of memory

0 BGP filter-list cache entries using 0 bytes of memory

BGP using 4920 total bytes of memory

BGP activity 12/0 prefixes, 28/0 paths, scan interval 60 secs

Neighbor        V          AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

10.1.2.2        4         500       8       8        9    0    0 00:02:42  4

10.1.3.3        4         300       8       8        9    0    0 00:02:12  4

10.1.3.130      4         300       8       8        9    0    0 00:02:11  4

Use the show bgp ipv4 unicast commands to view the specified BGP tables. Note that R1 has multiple paths to each destination network. Take note of the next hop address for the destination networks marked with the “>” symbol.

R1# show bgp ipv4 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>   192.168.1.0/27   0.0.0.0                  0         32768 i

*>   192.168.1.64/26  0.0.0.0                  0         32768 i

*    192.168.2.0/27   10.1.3.130                             0 300 500 i

*>                    10.1.2.2                 0             0 500 i

*                     10.1.3.3                               0 300 500 i

*    192.168.2.64/26  10.1.3.130                             0 300 500 i

*>                    10.1.2.2                 0             0 500 i

*                     10.1.3.3                               0 300 500 i

*    192.168.3.0/27   10.1.3.130               0             0 300 i

*                     10.1.2.2                               0 500 300 i

*>                    10.1.3.3                 0             0 300 i

*    192.168.3.64/26  10.1.3.130               0             0 300 i

*                     10.1.2.2                               0 500 300 i

*>                    10.1.3.3                 0             0 300 i

Use the show ip route bgp commands to view the routing tables. Note that there is only one route to each destination, and that the routes included in the routing table have the same next hop as those with the “>” symbol in the BGP tables.

R1# show ip route bgp | begin Gateway

Gateway of last resort is not set

192.168.2.0/24 is variably subnetted, 2 subnets, 2 masks

B        192.168.2.0/27 [20/0] via 10.1.2.2, 00:04:10

B        192.168.2.64/26 [20/0] via 10.1.2.2, 00:04:10

192.168.3.0/24 is variably subnetted, 2 subnets, 2 masks

B        192.168.3.0/27 [20/0] via 10.1.3.3, 00:04:09

B        192.168.3.64/26 [20/0] via 10.1.3.3, 00:04:09

Close configuration window

## Configure and Verify BGP Path Manipulation Settings on all Routers

In Part 3, you will configure path manipulation tools for BGP. The way these tools are being used here is not meant to represent best practice, but to assess your ability to complete the required configurations.

### Configure ACL-based route filtering.

In this step, you will configure R3 so that it only sends ASN300 networks to R1; it will not tell R1 that it knows about the networks in ASN200.

On R1, issue the command show bgp ipv4 unicast | i 300 to see what prefixes ASN300 is sharing via BGP. Take note of those prefixes that do not originate in ASN300.

Open configuration window

R1# show bgp ipv4 unicast | i 300

*    192.168.2.0/27   10.1.3.3                               0 300 500 i

*                     10.1.3.130                             0 300 500 i

*    192.168.2.64/26  10.1.3.3                               0 300 500 i

*                     10.1.3.130                             0 300 500 i

*    192.168.3.0/27   10.1.2.2                               0 500 300 i

*>                    10.1.3.3                 0             0 300 i

*                     10.1.3.130               0             0 300 i

*    192.168.3.64/26  10.1.2.2                               0 500 300 i

*>                    10.1.3.3                 0             0 300 i

*                     10.1.3.130               0             0 300 i

On R3, configure an access list designed to match the source address and mask of the networks belonging to ASN300:

R3(config)# ip access-list extended ALLOWED_TO_R1

R3(config-ext-nacl)# permit ip 192.168.3.0 0.0.0.0 255.255.255.224 0.0.0.0

R3(config-ext-nacl)# permit ip 192.168.3.64 0.0.0.0 255.255.255.192 0.0.0.0

R3(config-ext-nacl)# exit

On R3, apply the ALLOWED_TO_R1 ACL as a distribute list to the IPv4 neighbor adjacencies with R1.

R3(config)# router bgp 300

R3(config-router)# address-family ipv4 unicast

R3(config-router-af)# neighbor 10.1.3.1 distribute-list ALLOWED_TO_R1 out

R3(config-router-af)# neighbor 10.1.3.129 distribute-list ALLOWED_TO_R1 out

R3(config-router-af)# end

Perform a reset of the IPv4 adjacency with R1 for the outbound traffic without tearing down the session.

R3# clear bgp ipv4 unicast 6500 out

On R1, issue the command show bgp ipv4 unicast | i 300 to see what prefixes routes ASN300 is now sharing via BGP. All of the prefixes should now originate in ASN300:

R1# show bgp ipv4 unicast | i 300

*    192.168.3.0/27   10.1.2.2                               0 500 300 i

*>                    10.1.3.3                 0             0 300 i

*                     10.1.3.130               0             0 300 i

*    192.168.3.64/26  10.1.2.2                               0 500 300 i

*>                    10.1.3.3                 0             0 300 i

*                     10.1.3.130               0             0 300 i

Close configuration window

### Configure prefix-list-based route filtering.

In this step, you will configure R1 so that it only accepts ASN500 networks from R2; it will not accept information about ASN300 networks from R2.

On R1, issue the command show bgp ipv4 unicast | begin 192.168.3 to see what prefixes ASN500 is sharing via BGP. Take note of those prefixes that do not originate in ASN500.

Open configuration window

R1# show bgp ipv4 unicast | begin 192.168.3

*    192.168.3.0/27   10.1.3.130               0             0 300 i

*                     10.1.2.2                               0 500 300 i

*>                    10.1.3.3                 0             0 300 i

*    192.168.3.64/26  10.1.3.130               0             0 300 i

*                     10.1.2.2                               0 500 300 i

*>                    10.1.3.3                 0             0 300 i

On R1, configure a prefix list designed to match the source address and mask of networks belonging to ASN500.

R1(config)# ip prefix-list ALLOWED_FROM_R2 seq 5 permit 192.168.2.0/24 le 27

Apply the ALLOWED_FROM_R2 prefix list to the IPv4 neighbor adjacencies for R2.

R1(config)# router bgp 6500

R1(config-router)# address-family ipv4 unicast

R1(config-router-af)# neighbor 10.1.2.2 prefix-list ALLOWED_FROM_R2 in

R1(config-router-af)# end

Perform a reset of the IPv4 adjacency with R2 for the inbound traffic without tearing down the session.

R1# clear bgp ipv4 unicast 500 in

On R1, issue the command show bgp ipv4 unicast | i 500 to see what prefixes routes ASN500 is now sharing via BGP. All of the prefixes should now originate in ASN500.

R1# show bgp ipv4 unicast | i 500

*>   192.168.2.0/27   10.1.2.2                 0             0 500 i

*>   192.168.2.64/26  10.1.2.2                 0             0 500 i

Close configuration window

### Configure an AS-PATH ACL to filter routes being advertised.

In this step, you will configure R1 so that it only sends ASN6500 networks to R2; it will not forward information about prefixes from any other ASN to ASN500.

On R2, issue the command show bgp ipv4 unicast | begin Network to see what prefixes ASN6500 is sharing via BGP. Take note of those prefixes that do not originate in ASN6500. Advertising these routes could set ASN6500 up as a transit AS, and that is not a desirable scenario.

Open configuration window

R2# show bgp ipv4 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*    192.168.1.0/27   10.2.3.3                               0 300 6500 i

*>                    10.1.2.1                 0             0 6500 i

*    192.168.1.64/26  10.2.3.3                               0 300 6500 i

*>                    10.1.2.1                 0             0 6500 i

*>   192.168.2.0/27   0.0.0.0                  0         32768 i

*>   192.168.2.64/26  0.0.0.0                  0         32768 i

*    192.168.3.0/27   10.1.2.1                               0 6500 300 i

*>                    10.2.3.3                 0             0 300 i

*    192.168.3.64/26  10.1.2.1                               0 6500 300 i

*>                    10.2.3.3                 0             0 300 i

On R1, configure AS-PATH ACL to match the routes from the local ASN.

R1(config)# ip as-path access-list 1 permit ^$

On R1, apply the AS-PATH ACL as a filter-list on the adjacency configured with R2.

R1(config)# router bgp 6500

R1(config-router)# address-family ipv4 unicast

R1(config-router-af)# neighbor 10.1.2.2 filter-list 1 out

R1(config-router-af)# end

On R1, perform a reset of the IPv4 adjacency with R2 for the outbound traffic without tearing down the session.

R1# clear bgp ipv4 unicast 500 out

On R2, issue the command show bgp ipv4 unicast | i 6500 to see what prefixes routes ASN6500 is now sharing via BGP. All of the prefixes should now originate in ASN6500.

R2# show bgp ipv4 unicast | i 6500

*    192.168.1.0/27   10.2.3.3                               0 300 6500 i

*>                    10.1.2.1                 0             0 6500 i

*    192.168.1.64/26  10.2.3.3                               0 300 6500 i

*>                    10.1.2.1                 0             0 6500 i

Close configuration window

Close configuration window

### Configure BGP path attribute manipulation to effect routing.

In this step, you will configure R1 so that it prefers the next-hop address of 192.168.3.130 over 192.168.3.3, which would normally be the preferred path to ASN300 networks. You will do this by using a prefix list to identify the destination networks and then use a route map to match the prefix list and set the matched networks to have a local preference of 250.

On R1, issue the command show ip route bgp and take note of the next hop addresses for the 192.168.3.0/27 and 192.168.3.64/26 networks. Then issue the command show bpg ipv4 unicast and note that the 10.1.3.130 is a valid next hop (It’s just not the best next hop, according to the BGP path selection algorithm.) Lastly, issue the command show bgp ipv4 unicast 192.168.3.0 to see details about all the paths available and which one was selected.

Open configuration window

R1# show bgp ipv4 unicast 192.168.3.0

BGP routing table entry for 192.168.3.0/27, version 8

Paths: (2 available, best #1, table default)

Advertised to update-groups:

1

Refresh Epoch 1

300

10.1.3.3 from 10.1.3.3 (3.3.3.3)

Origin IGP, metric 0, localpref 100, valid, external, best

rx pathid: 0, tx pathid: 0x0

Refresh Epoch 1

300

10.1.3.130 from 10.1.3.130 (3.3.3.3)

Origin IGP, metric 0, localpref 100, valid, external

rx pathid: 0, tx pathid: 0

On R1, configure a prefix list designed to match the source address and mask of networks belonging to ASN300.

R1(config)# ip prefix-list PREFERRED_IPV4_PATH seq 5 permit 192.168.3.0/24 le 27

Create a route-map named USE_THIS_PATH_FOR_IPV4 that matches on the prefix list you just created and sets the local preference to 250.

R1(config)# route-map USE_THIS_PATH_FOR_IPV4 permit 10

R1(config)# match ip address prefix-list PERFERRED_IPV4_PATH

R1(config)# set local-preference 250

Next, apply this route map to the BGP neighbor 10.1.3.130.

R1(config)# router bgp 6500

R1(config-router)# address-family ipv4 unicast

R1(config-router-af)# neighbor 10.1.3.130 route-map USE_THIS_PATH_FOR_IPV4 in

R1(config-router-af)# end

Perform a reset of the IPv4 adjacency with R3 for the inbound traffic without tearing down the session.

R1# clear bgp ipv4 unicast 300 in

On R1, issue the command show ip route bgp and take note of the next hop addresses for the 192.168.3.0/27 and 192.168.3.64/26 networks; it should be 10.1.3.130 for both. Issue the command show bgp ipv4 unicast and you should see the local preference value in the appropriate column.

R1# show ip route bgp | begin Gateway

Gateway of last resort is not set

192.168.2.0/24 is variably subnetted, 2 subnets, 2 masks

B        192.168.2.0/27 [20/0] via 10.1.2.2, 00:35:17

B        192.168.2.64/26 [20/0] via 10.1.2.2, 00:35:17

192.168.3.0/24 is variably subnetted, 2 subnets, 2 masks

B        192.168.3.0/27 [20/0] via 10.1.3.130, 00:00:08

B        192.168.3.64/26 [20/0] via 10.1.3.130, 00:00:08

R1# show bgp ipv4 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>   192.168.1.0/27   0.0.0.0                  0         32768 i

*>   192.168.1.64/26  0.0.0.0                  0         32768 i

*>   192.168.2.0/27   10.1.2.2                 0             0 500 i

*>   192.168.2.64/26  10.1.2.2                 0             0 500 i

*    192.168.3.0/27   10.1.3.3                 0             0 300 i

*>                    10.1.3.130               0    250      0 300 i

*    192.168.3.64/26  10.1.3.3                 0             0 300 i

*>                    10.1.3.130               0    250      0 300 i

Close configuration window