# Lab - Configure Route Redistribution Between EIGRP and OSPF Topology

# Addressing Table

| Device | Interface | IPv4 Address/Mask |
| --- | --- | --- |
| R1 R1 R1 | E0/0 | 10.1.12.1/24 |
|  | E0/1 | 10.1.11.1/24 |
|  | Loopback 0 | 10.1.1.1/24 |
| R2 R2 | E0/0 | 10.1.12.2/24 |
|  | E0/1 | 10.1.23.2/24 |
| R3 R3 R3 | E0/0 | 10.1.23.3/24 |
|  | E0/1 | 10.1.32.1/24 |
|  | Loopback 0 | 10.3.3.3/24 |
| D1 D1 | E2/0 | 10.1.11.2/24 |
|  | Loopback 0 | 209.165.201.1/25 |
| D2 D2 | E2/0 | 10.1.32.2/24 |
|  | Loopback 0 | 198.51.100.1/25 |

# Objectives

Part 1: Build the Network and Configure Basic Device Settings

Part 2: Verify OSPFv3 AF Neighborships and Routing for IPv4

Part 3: Verify EIGRP Neighborships and Routing for IPv4

Part 4: Configure Redistribution from OSPFv3 to EIGRP

Part 5: Configure Redistribution from EIGRP to OSPFv3

# Background / Scenario

In this lab, you will configure redistribution from OSPF into EIGRP for IPv4, and redistribution of EIGRP into OSPF for IPv4. You will also change the metric type for EIGRP routes redistributed into OSPF.

D1, R1 and R2 are configured with OSPFv3 for IPv4 address family, while R2, R3 and D2 are configured with EIGRP using named mode for IPv4 address family.

Note: This lab is an exercise in configuring and verifying two-way route redistribution on R2. Route redistribution in this lab does not reflect networking best practices.

Note: The routers used with CCNP hands-on labs are Cisco 4221 with Cisco IOS XE Release 16.9.4 (universalk9 image). The switches used in the labs are Cisco Catalyst 3650 with Cisco IOS XE Release 16.9.4 (universalk9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs.

Note: Make sure that all the devices have been erased and have no startup configurations. If you are unsure, contact your instructor.

Instructor Note: Refer to the Instructor Lab Manual for the procedures to initialize and reload devices.

# Required Resources

3 Routers (Cisco 4221 with Cisco IOS XE Release 16.9.4 universal image or comparable)

2 Switches (Cisco 3650 with Cisco IOS XE release 16.9.4 universal image or comparable)

1 PC (Choice of operating system with terminal emulation program installed)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

# Instructions

## Build the Network and Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings.

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

### Configure basic settings for each device.

Console into each device, enter global configuration mode, and apply the basic settings for the lab. Initial configurations for each device are listed below.

Open configuration window

Router R1

hostname R1

no ip domain lookup

ipv6 unicast-routing

banner motd # R1, Configure Route Redistribution Between EIGRP and OSPF #

line con 0

exec-timeout 0 0

logging synchronous

exit

router ospfv3 1

router-id 1.1.1.1

exit

interface e0/0

ip address 10.1.12.1 255.255.255.0

ipv6  enable

ospfv3 1 ipv4 area 0

no shutdown

exit

interface e0/1

ip address 10.1.11.1 255.255.255.0

ipv6  enable

ospfv3 1 ipv4 area 11

no shutdown

exit

interface loopback 0

ip address 10.1.1.1 255.255.255.0

ospfv3 network point-to-point

ipv6  enable

ospfv3 1 ipv4 area 0

no shutdown

exit

router ospfv3 1

address-family ipv4 unicast

passive-interface Loopback0

exit-address-family

end

Router R2

hostname R2

no ip domain lookup

ipv6 unicast-routing

banner motd # R2, Configure Route Redistribution Between EIGRP and OSPF #

line con 0

exec-timeout 0 0

logging synchronous

exit

router ospfv3 1

router-id 2.2.2.2

address-family ipv4 unicast

exit-address-family

interface e0/0

ip address 10.1.12.2 255.255.255.0

ipv6  enable

ospfv3 1 ipv4 area 0

no shutdown

exit

interface e0/1

ip address 10.1.23.2 255.255.255.0

no shutdown

exit

router eigrp CISCO

address-family ipv4 unicast autonomous-system 64512

af-interface default

shutdown

exit-af-interface

af-interface Ethernet0/1

no shutdown

exit-af-interface

topology base

exit-af-topology

network 10.1.23.0 0.0.0.255

eigrp router-id 2.2.2.2

exit-address-family

end

Router R3

hostname R3

no ip domain lookup

ipv6 unicast-routing

banner motd # R3, Configure Route Redistribution Between EIGRP and OSPF #

line con 0

exec-timeout 0 0

logging synchronous

exit

interface e0/0

ip address 10.1.23.3 255.255.255.0

no shutdown

exit

interface e0/1

ip address 10.1.32.3 255.255.255.0

no shutdown

exit

interface loopback 0

ip address 10.3.3.3 255.255.255.0

no shutdown

exit

router eigrp CISCO

address-family ipv4 unicast autonomous-system 64512

af-interface default

shutdown

exit-af-interface

af-interface Ethernet0/0

no shutdown

exit-af-interface

af-interface Ethernet0/1

no shutdown

exit-af-interface

af-interface Loopback0

no shutdown

exit-af-interface

topology base

exit-af-topology

network 10.1.23.0 0.0.0.255

network 10.1.32.0 0.0.0.255

network 10.3.3.0 0.0.0.255

eigrp router-id 3.3.3.3

exit-address-family

end

Switch D1

hostname D1

no ip domain lookup

ip routing

ipv6 unicast-routing

banner motd # D1, Configure Route Redistribution Between EIGRP and OSPF #

line con 0

exec-timeout 0 0

logging synchronous

exit

router ospfv3 1

router-id 11.11.11.11

exit

interface range e0/0-3,e1/0-3,e2/0-3

shutdown

exit

interface e2/0

no switchport

ip address 10.1.11.2 255.255.255.0

ipv6  enable

ospfv3 1 ipv4 area 11

no shutdown

exit

interface loopback 0

ip address 209.165.201.1 255.255.255.128

no shutdown

exit

router ospfv3 1

address-family ipv4 unicast

passive-interface Loopback0

default-information originate

exit-address-family

ip route 0.0.0.0 0.0.0.0 Loopback0

exit

Switch D2

hostname D2

no ip domain lookup

ip routing

ipv6 unicast-routing

banner motd # D2, Configure Route Redistribution Between EIGRP and OSPF #

line con 0

exec-timeout 0 0

logging synchronous

exit

interface range e0/0-3,e1/0-3,e2/0-3

shutdown

exit

interface e2/0

no switchport

ip address 10.1.32.2 255.255.255.0

no shutdown

exit

interface loopback 0

ip address 198.51.100.1 255.255.255.128

no shutdown

router eigrp CISCO

address-family ipv4 unicast autonomous-system 64512

af-interface default

shutdown

exit-af-interface

af-interface Loopback0

no shutdown

passive-interface

exit-af-interface

af-interface Ethernet2/0

no shutdown

exit-af-interface

topology base

exit-af-topology

network 10.1.32.0 0.0.0.255

network 198.51.100.0 0.0.0.127

eigrp router-id 22.22.22.22

exit-address-family

exit

Save the running configuration to startup-config on all devices.

Close configuration window

## Verify OSPFv3 AF Neighborships and Routing for IPv4

In this part, you will verify that OSPF has established neighbor relationships and routing for IPv4.

### Verify OSPFv3 AF neighborships on R1.

Verify R1 has OSPFv3 neighbors: two neighbors from IPv4 address family

Open configuration window

R1# show ospfv3 neighbor

OSPFv3 1 address-family ipv4 (router-id 1.1.1.1)

Neighbor ID     Pri   State           Dead Time   Interface ID    Interface

2.2.2.2           1   FULL/BDR        00:00:36    6               Ethernet0/0

11.11.11.11       1   FULL/BDR        00:00:31    38              Ethernet0/1

The output shows four OSPFv3 neighbors: two neighbors from IPv4 address family.

Close configuration window

### Verify the IPv4 OSPFv3 routing table on R2.

Verify the OSPFv3 IPv4 routing table on R2. Notice the default route, the intra–area, and inter–area OSPF routes are installed and received from 10.1.12.1, which is R1.

Open configuration window

R2# show ip route ospfv3 | begin Gateway

Gateway of last resort is 10.1.12.1 to network 0.0.0.0

O*E2  0.0.0.0/0 [110/1] via 10.1.12.1, 02:41:28, Ethernet0/0

10.0.0.0/8 is variably subnetted, 8 subnets, 2 masks

O        10.1.1.0/24 [110/1] via 10.1.12.1, 02:49:12, Ethernet0/0

O IA     10.1.11.0/24 [110/2] via 10.1.12.1, 02:44:58, Ethernet0/0

From R2, ping the Loopback 0 address on D1. The ping should be successful.

R2# ping 209.165.201.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.201.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/2 ms

Close configuration window

## Verify EIGRP Neighborships and Routing for IPv4

In this part, you will verify that EIGRP has established neighbor relationships and routing for IPv4.

### Verify EIGRP for IPv4 neighborships on R3.

Issue the command to verify EIGRP has two IPv4 neighbors, as shown.

Open configuration window

R3# show ip eigrp neighbors

EIGRP-IPv4 VR(CISCO) Address-Family Neighbors for AS(64512)

H   Address                 Interface              Hold Uptime   SRTT   RTO  QSeq

(sec)         (ms)       CntNum

1   10.1.32.2               Gi0/0/1                  10 20:13:56    3   100  013

0   10.1.23.2               Gi0/0/0                  13 20:31:08    1   100  019

Notice the two IPv4 neighbors, 10.1.23.2 and 10.1.32.2.

Close configuration window

### Verify EIGRP for IPv4 routing table on R2.

Issue the command to display the EIGRP IPv4 routing table on R2, as shown.

Open configuration window

R2# show ip route eigrp | begin 10.0

10.0.0.0/8 is variably subnetted, 8 subnets, 2 masks

D        10.1.32.0/24 [90/15360] via 10.1.23.3, 20:35:38, Ethernet0/1

D        10.3.3.0/24 [90/10880] via 10.1.23.3, 20:44:06, Ethernet0/1

198.51.100.0/25 is subnetted, 1 subnets

D        198.51.100.0 [90/16000] via 10.1.23.3, 20:29:04, Ethernet0/1

Notice three internal EIGRP routes from 10.1.23.3, which is R3.

## Configure Redistribution from OSPFv3 to EIGRP

Recall that every protocol provides a seed metric at the time of redistribution. By default, when source protocols, such as, OSPF, RIP, and IS-IS, are redistributed into EIGRP, they are given an administrative distance of 170 and a seed metric of infinity. This prevents the installation of the redistributed routes into the EIGRP topology table. The seed metric can be set using the redistribute or default-metric command. Additionally, when using a route map, the seed metric can be configured using the set metric option.

For IPv4, you will set the seed metric using the redistribute command and the default-metric command.

### Redistribute OSPFv3 into EIGRP for IPv4.

In this step were going to the destination EIGRP AS 64512 to perform redistribution. Since EIGRP is using named mode the redistribute command is entered in the address family topology configuration mode, as shown.

R2(config)# router eigrp CISCO

R2(config-router)# address-family ipv4 autonomous-system 64512

R2(config-router-af)# topology base

R2(config-router-af-topology)# redistribute ospfv3 1 metric 1000000 10 255 1 1500

R2(config-router-af-topology)# end

close configuration window

### On D2, verify redistribution of OSPFv3.

Issue the show ip route eigrp on D2 to see the external EIGRP routes from OSPFv3.

Open configuration window

D2# show ip route eigrp | begin Gateway

Gateway of last resort is 10.1.32.3 to network 0.0.0.0

D*EX  0.0.0.0/0 [170/66560] via 10.1.32.3, 00:03:59, Ethernet2/0

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

D EX     10.1.1.0/24

[170/66560] via 10.1.32.3, 00:03:59, Ethernet2/0

D EX     10.1.11.0/24

[170/66560] via 10.1.32.3, 00:03:59, Ethernet2/0

D EX     10.1.12.0/24

[170/66560] via 10.1.32.3, 00:03:59, Ethernet2/0

D        10.1.23.0/24

[90/15360] via 10.1.32.3, 21:20:07, Ethernet2/0

D        10.3.3.0/24 [90/10880] via 10.1.32.3, 21:20:07, Ethernet2/0

close configuration window

Notice the gateway of last resort has been set and D2 has learned four external EIGRP routes which originated from OSPFv3. The OSPFv3 routes are imported into EIGRP as external, D EX routes with an administrative distance of 170, which are higher than the internal EIGRP routes of 90.

close configuration window

## Configure Redistribution from EIGRP for IPv4 into OSPFv3

In this part, you will perform EIGRP for IPv4 redistribution into OSPFv3.

Note: When redistributing into OSPFv2, you must include the subnets keyword. The keyword subnets is required for classless networks to be advertised. If omitted only classful networks using a classful mask will be redistributed.

### On R2, redistribute EIGRP into OSPFv3.

The redistribute command is always performed on the destination protocol. Start by accessing the OSPFv3 address family for IPv4. Then redistribute the source protocol, EIGRP 64512 into the destination protocol, as shown.

Open configuration window

R2(config)#

R2(config-router)#

R2(config-router-af)#

Note: Do not leave AF configuration mode.

### Verify redistribution on D1.

Issue the show ip route ospfv3 on D1 to see the external OSPF routes from EIGRP.

Open configuration window

D1# show ip route ospfv3 | begin Gateway

Gateway of last resort is 0.0.0.0 to network 0.0.0.0

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

O IA     10.1.1.0/24 [110/2] via 10.1.11.1, 02:52:36, Ethernet2/0

O IA     10.1.12.0/24 [110/2] via 10.1.11.1, 1d01h, Ethernet2/0

O E2     10.1.23.0/24 [110/20] via 10.1.11.1, 00:03:55, Ethernet2/0

O E2     10.1.32.0/24 [110/20] via 10.1.11.1, 00:03:55, Ethernet2/0

O E2     10.3.3.0/24 [110/20] via 10.1.11.1, 00:03:55, Ethernet2/0

198.51.100.0/25 is subnetted, 1 subnets

O E2     198.51.100.0 [110/20] via 10.1.11.1, 00:03:55, Ethernet2/0

close configuration window

Notice the highlighted external E2 OSPF routes. By default, external LSAs appear in the routing table marked as E2 with an external cost of 20.

### Redistribute EIGRP into OSPFv3 using a Type 1.

From the address family configuration mode, modify the redistribute command configured in Step 1 to specify an external type 1.

R2(config-router-af)# redistribute eigrp 64512 metric-type ?

1  Set OSPF External Type 1 metrics

2  Set OSPF External Type 2 metrics

R2(config-router-af)#

R2(config-router-af)# exit

close configuration window

### Verify redistribution again on D1.

Issue the show ip route ospfv3 on D1 to see the external OSPF routes.

Open configuration window

D1# show ip route ospfv3 | begin Gateway

Gateway of last resort is 0.0.0.0 to network 0.0.0.0

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

O IA     10.1.1.0/24 [110/2] via 10.1.11.1, 03:10:29, Ethernet2/0

O IA     10.1.12.0/24 [110/2] via 10.1.11.1, 1d02h, Ethernet2/0

O E1     10.1.23.0/24 [110/22] via 10.1.11.1, 00:10:11, Ethernet2/0

O E1     10.1.32.0/24 [110/22] via 10.1.11.1, 00:10:11, Ethernet2/0

O E1     10.3.3.0/24 [110/22] via 10.1.11.1, 00:10:11, Ethernet2/0

198.51.100.0/25 is subnetted, 1 subnets

O E1     198.51.100.0 [110/22] via 10.1.11.1, 00:10:11, Ethernet2/0

Notice the highlighted external E1 OSPF routes. These E1 routes have a cost of 22 which includes the default cost of 20 plus the internal cost of 2.

Close configuration window

From D2 ping the Loopback address on D1 using Loopback address of D2. The ping should be successful. This verifies successful two-way redistribution on R2 and end-to-end connectivity for IPv4.

Open configuration window

- D2# ping 209.165.201.1 source loopback 0

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.201.1, timeout is 2 seconds:

Packet sent with a source address of 198.51.100.1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 2/2/3 ms

Close configuration window

Close configuration window

close configuration window

# Reflection Questions

What is the difference between an external OSPF E2 and E1?

OSPF metric type E2 is the default, if not specified. An E2 has an external cost of 20. Metric type 1 or E1 calculates the cost by adding the external cost to the internal cost for each link the packet traverses. Use an E1 when there are multiple ASBRs advertising an external route to the same AS.

What are three ways to set a seed metric during redistribution?

Using the redistribute command, using the default-metric command and using a route map with the set metric command

What is the default action in a route map if you do not include the match command?

The default action is to match any